МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 1

1. Translate the following text into Ukrainian.

The pump

In a reciprocating pump a piston (or plunger) moves up and down and forces the liquid in and out  of the pump chamber by creating a vacuum and a pressure force.

In a single acting reciprocating pump the liquid   is drawn in when the piston goes up, and is forced out when the piston goes down.

In a  double-acting reciprocating pump the upward and downward motions of the  piston cause  simultaneous suction- and  discharge actions.

Double-acting displacement pump

When the piston goes down the lower chamber is emptied through the discharge valve (D2) by   pressure. The other discharge valve (D1) is closed by suction.

The suction valve (S1) is opened by suction, allowing liquid to enter the upper pump chamber. The other suction valve (S2) is closed by pressure. When the piston goes up, the reverse process will take place.

In a gearwheel pump the liquid (fuel oils and lubricating oil) enters through the suction side.

The liquid is then captured between the teeth of the gearwheels (cogs) and is carried around towards the discharge side.

The centrifugal pump only works when it has been immersed in the liquid. In its stationary casing a rotating impeller causes a centrifugal force that will discharge the liquid at high velocity through the discharge line.

___30

2. Complete  the sentences below with information from the text

1) In a single acting reciprocating pump the liquid   is……… when the piston goes up, and is forced out when the piston …

2) The liquid is then captured between the teeth of the gearwheels (cogs) and is ……………

3) In its stationary casing a rotating impeller causes a centrifugal force  …………..

___10

3.Read the text again and find information to answer these questions.

1. What do the upward and downward motions of the piston cause in a  double-acting reciprocating pump ?

2. When does the centrifugal pump only work?

3. How does a piston (or plunger) force the liquid in and out of the pump chamber  in a reciprocating pump?

4.  How does the liquid (fuel oils and lubricating oil) enter in a gearwheel pump?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) Pipes should be led to prevent water lodgement.

2) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

3) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

4) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

5) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

- In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 2

1. Translate the following text into Ukrainian.

The exhaust gas boil

The effective power obtained from heavy fuel oil (HFO) is about 50%   (provided that both engine and engineers perform their tasks well).

This means a heavy loss of energy, which for the major part is wasted in the exhaust gases through the flue and funnel.

These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through an exhaust gas boiler.

The position of the exhaust gas boiler, or waste heat boiler, may be in the engine room or in the funnel.

The idea is quite simple: instead of letting the exhaust gases escape from the engine through the flue and funnel, the gases are first passed through a boiler.

The type of boiler that is used for this purpose is often a so called fire tube boiler.

It has a steam chamber in its hemispherical top.

The boiler shell consists of steel plates that are welded together.

To prevent any thermal loss and to protect the boiler shell from being damaged by the heat, fire bricks are used.

Exhaust gases enter the boiler through the exhaust gas inlet. The exhaust gases heat up banks of fire tubes, which heat up the water in the boiler. The exhaust gases then leave the boiler through the exhaust gas outlet.

The water in the boiler is turned to steam, which may be used for heating purposes. On large vessels the steam may even be used to drive a turbo-generator that will produce electricity that can be used to drive auxiliary engines or for the lighting system.

The exhaust gas boiler is often fitted with an oil-firing installation. In such a case the boiler is called a compound boiler, which means that it can be run both on exhaust gases and by oil firing when the ship is in port and the idle engine does not produce any exhaust gases.

___30

2. Complete  the sentences below with information from the text

1) These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through …

2) The boiler shell consists of …

3) The water in the boiler is turned to …

___10

3. Read the text again and find information to answer these questions.

1. What does a  fire tube boiler have?

2.  What is used to protect the boiler shell from being damaged by the heat?

3.  What do exhaust gases enter the boiler through ?

4.  What do the exhaust gases  leave the boiler through?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

2) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their facesparallel.

3) When the pump is not up to proper speed, the discharge valve should be opened.

4) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

5) Pipes should be led to prevent water lodgement.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain how the steering engine work? _______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 3

1. Translate the following text into Ukrainian.

Electric motors

Aboard vessels the electric motors and hydraulic systems perform most of the hard labour. They are indispensable in daily operations. The maintenance and overhauling of electric motors and appliances is therefore of the utmost importance.

Electric motors and generators are similar in construction.

The main components of an electric motor are the rotor and the stator (or armature).

In an electric motor the rotary motion of the rotor within the stationary stator is employed to actuatepumps, winches, derricks, etc.

Most electric motors are run on alternating current (A/C).

The old-fashioned compound motor combines the advantages of the even more old fashioned  shunt motor and the series motor. The compound motor has a high starting torque and a constant running speed.

Nowadays we know two main types of electric motors: the a-synchronous induction motor, which is widely used aboard ships, and the synchronous induction motor, which is hardly used aboard ships.

The advantage of a-synchronous induction motor is the lack of vulnerable carbon brushes that require frequent maintenance.

Its disadvantages are the large amount of initial current that it requires, and the low starting torque it produces.

In the synchronous induction motor three-phase alternating current, either 220, 380 or 440 volts, passes through coils of copper wire in the stator, creating an alternating magnetic field.

Carbon brushes convey the current to the rotor, which will start to rotate.

The advantages of the synchronous motor are the small amount of initial current it requires, and its high starting torque. However, the synchronous motor is expensive, and the carbon brushes require frequent maintenance.

___30

2. Complete  the sentences below with information from the text

1) The advantage of a-synchronous induction motor is the lack of …

2) Carbon brushes convey the current to the ….

3) The compound motor has a high starting ….

___10

3.Read the text again and find information to answer these questions.

1. What are the main components of an electric motor?

2. What are  two main types of electric motors known nowadays?

3. What are the advantages of the synchronous motor?

4. What is the principle difference between a-synchronous induction motor and the synchronous induction motor?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

2) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

3) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

4) When the pump is not up to proper speed, the discharge valve should be opened.

5) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

- See that the emergency and throttle valves are set open.

- Open all the drains.

- Seal the glands by opening steam valve, if provided. Start condensing plant.

- Open the stop valve slightly, sufficiently to warm the turbine without starting it.

- Open the water connection valves. See also that the cooler is not by-passed if a by-pass is fitted.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _____________

Explain the work of the anchor winch. _________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 4

1. Translate the following text into Ukrainian.

Diesel engines

A Diesel engine is a propulsion plant in which the heat that is generated by internal combustion of a mixture of fuel and air is converted into power.

There are slow speed -, medium speed - and high-speed Diesel engines.

By "speed" is understood the number of revolutions per minute (RPM) the shaft can make.

The slow-speed Diesel engine is applied in large vessels with large engine rooms. These engines are very reliable, but take up a lot of space.

Slow-speed Diesel engines consume Heavy Fuel Oil (HFO), which is an inexpensive fuel. This type of fuel, also called residual oil, is used in medium-speed Diesel engines, too. These engines are favoured in large vessels, e.g. in ferries. Compared to the slow-speed engine the medium-speed engine is more compact. The medium-speed Diesel engine requires more maintenance.

High speed Diesel engines are used in small, highly manoeuvrable vessels. The rated capacity of such an engine is very high, but so is its fuel consumption.

Diesel engines may have cylinders that are placed "in line" or in a bevel (oblique) position. Compared to the in-line engine the advantages of the V-engine are obvious: a V-engine requires only one crankshaft and takes up less space. Therefore the V-engine is very suited to be installed in engine rooms of limited size, contrary to the in-line engine, which cannot be used in small engine rooms due to its height. The in-line engine is not as costly as the V-engine. The construction of the in-line engine is simple, which makes the maintenance and overhauling relatively easy and cheap.

As to the construction of Diesel engines we distinguish two main types: the trunk engine and the crosshead engine. In a trunk engine the piston pin and crank pin are directly connected to each other by the connecting rod. Trunk engines are of compacted build and are mostly medium speed- or high speed engines. Compared to the crosshead engines they do require more maintenance. The crosshead engine is taller because of the crosshead and piston rod, which require more space. This engine is mainly used  in  large vessels.The crosshead serves as a hinging connection between piston rod and connecting rod.

___30

2. Complete  the sentences below with information from the text

1) By “speed” is understood the number of …

2) The construction of the in-line engine is simple, which makes the maintenance…

3) In a trunk engine the piston pin and crank pin are directly …

___10

3. Find the answers to these questions in the text.

1. What type of fuel Slow-speed Diesel engines consume?

2. Where are mostly used High speed Diesel engines are used ?

3. What are the advantages of V-engine?

4. What is the principle difference between trunk and crosshead engines?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) When the pump is not up to proper speed, the discharge valve should be opened.

2 After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

3) Pipes should be led to prevent water lodgement.

4) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

5) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ____________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 5

1. Translate the following text into Ukrainian.

Two-stroke engines and four-stroke engines

The piston in a cylinder makes a reciprocating motion - it goes up and down. The up and down motions of a piston are called strokes. The number of strokes of a piston that form a cycle in a cylinder is two or four. The two-stroke principle is applied in slow speed engines.

Compared with the four-stroke engine, its advantage is that each cylinder is capable of generating more power, because the number of power strokes by the piston is twice as many as with the four-stroke cycle at the same crankshaft speed.

The disadvantage of the two-stroke engine is that, unlike with the four-stroke engine, its effective power is limited due to the fact that its ability to burn fuel is less than in a four-stroke engine. This latter engine is used in smaller vessels with medium-speed or high-speed engines.

The two-stroke cycle begins when the piston is in its Bottom Dead Centre (BDC). Air enters the cylinder through the inlet ports. During the compression stroke the air in the cylinder is compressed while the piston travels upwards. Fuel is admitted to the cylinder at the end of this stroke, either mechanically by fuel pumps, or electrically by a common-rail system. The mixture of air and fuel ignites spontaneously due to the high compression and consequent high temperature in the cylinder.

During the power stroke the piston is pushed down by the "explosion" of the mixture in the cylinder. The reciprocating piston and connecting rod will cause the crankshaft to rotate. Now the camshaft is actuated, so that the rocking lever can open the valve (a pushrod may be inserted as a distance piece between camshaft and rocking lever).

Thus, at the end of the power stroke, the exhaust valves are opened, allowing the exhaust gases to escape into the exhaust gas manifold, after which the remaining exhaust gases are removed out of the cylinder by scavenging air. The scavenging system that is most widely employed nowadays is the uniflow scavenging  system,  because  it  removes  almost all  the  exhaust gases out of the  cylinder.

Cross scavenging- or loop scavenging systems are not effective enough and will leave behind small amounts of gases that will form carbon deposits in the cylinder. Before a new cycle is started, the exhaust valve is closed by the exhaust valve spring or by a hydraulic system.

___30

2.Complete  the sentences below with information from the text

1) Before a new cycle is started, the exhaust valve is closed by…

2) The reciprocating piston and connecting rod will cause …

3) Air enters the cylinder through …

___10

3. Read the text again and find information to answer these questions.

1. What does a stroke mean?

2. When is two-stroke principe applied?

3. What are advantages and disadvantages of two-stroke engines?

4.How are  gases  removed out of the cylinder ?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

- Pipes should be led to prevent water lodgement.

- It should be particularly noted that the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

- It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

- The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

- Horizontal pumps should be laid down upon steel or hard 
wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

4. In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

5. Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator______________________________

Explain the work of the steering engine ______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 6

1. Translate the following text into Ukrainian.

Generators

In order to provide electric power for the lighting system, auxiliaries, cranes, derricks, hatches and for domestic use, the ship is equipped with one or more generators.

Aboard vessels a generator produces electricity of 380 volts or 440 volts. The voltage can be stepped down (reduced) to 220 volts or even 110 volts by a transformer.

A generator produces either alternating current (A/C) or direct current (D/C). A converter is used to convert (change) A/C into D/C. Alternating current changes polarity about 50 times per second ("50 Hertz"). In the U.S.A. generators produce electricity of 60 Hz.

The main parts of a generator are the rotor, the stator and the commutator. The rotor is a rotating electro-magnet. It produces a magnetic field. The rotor rotates within the stator.

Around this stator, or armature, coils of wire are wound. These coils induce ("make") electric current. The process is called induction. The induced current is collected by the commutator. Carbon brushes pass the current to the circuit.

Nowadays the  modern generators do  not  have carbon  brushes  because they  require  regular maintenance. Instead, the rotor is excited by the direct current that is delivered by the stator.

___30

2. Complete  the sentences below with information from the text

1) The voltage can be stepped down (reduced) to 220 volts or even 110 volts by ____

2) A converter is used to ______

3) Instead, the rotor is excited by _______

___10

3. Read the text again and find information to answer these questions.

1. What kind of current does a generator produce?

2. What are the main parts of a generator?

3. Why don’t modern generators have carbon  brushes ?

4. Why is the ship equipped with one or more generators?

___10

4. Put the statements in the correct sequence.

- The system the piping soot main includes only the and turbo-generators steam the blowers.

- For operation many of supply auxiliary and saturated systems units of steam machinery systems steam the.

- Pressure maintained system at the auxiliary in the 15 is psig.

- Includes the condensers piping the system pumps and main auxiliary condensate and.

- System the piping feed the includes pump and feed the boiler main.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

5. In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. _________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 7

1. Translate the following text into Ukrainian.

Fuel Oil Systems

Boiler fuel oil Systems on board ship include oil tanks, fuel oil piping, fuel oil pumps, and the equipment used for heating, straining, measuring, and burning the fuel oil.

The fuel oil piping system includes (1) the fuel oil filling and transfer system, and (2) the fuel oil service system. The filling and transfer system is used for, receiving fuel oil and filling the fuel oil storage tanks, filling the fuel oil service tanks, changing the list of the ship by transferring fuel oil between port tanks and starboard tanks, changing the trim of the ship by transferring fuel oil between forward tanks and after tanks.

In general, the filling and transfer system consists of large mains running fore and aft, transfer mains, cross-connections, risers for taking on or discharging fuel oil, fuel oil booster and transfer pumps, lines and manifolds arranged so that the booster and transfer pumps can transfer oil from one tank to another and, when necessary, can deliver oil to the suction side of the fuel service pump.

The fuel oil service system consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

Main Lubricating Oil Systems

Main lubricating oil systems on steam-driven ships provide lubrication for the turbine bearings and the reduction gears. The system usually includes a filling and transfer system, a purifying system, and separate service system for the propulsion unit. On most ships, each lube oil service system includes three displacement pumps:

(1) a shaft-driven pump, attached to and driven by either the propulsion shaft or the quill shaft of the reduction gear, is used as the regular lube oil service pump when the shaft is turning fast enough so that the pump can supply the required oil pressure. (2) The turbine-driven lube oil service pump is used while the ship is getting under way, and is then used as standby at normal speeds. The motor-driven lube oil service pump serves as standby for the other two pumps.

___30

2. Find the answers to these questions in the text.

- What arrangements and equipment do the boiler fuel oil systems include?

- What is the function of the filling and transfer system?

- How can the ship's list or trim be corrected?

- What arrangements allow to transfer oil from one tank to another?

- Which side of a pump is called the suction side?

- What components does the fuel oil service system include?

- What is the English for the system providing lubrication for the turbine bearings?

- What type of pump is used in these systems?

- How are the lube oil pumps driven?

- Which pump is used when the ship is getting under way?

___10

3. Complete the sentences with the correct form of the vebr: to serve, to provide, to consist of, to be used for, to include.

Boiler fuel oil system on board ship _________ oil tanks, fuel oil piping, fuel oil pumps and other equipment. The filling and transfer system ____________ performing many functions. It __________ large mains, transfer mains, cross-connections, risers, fuel oil booster and transfer pumps, lines and manifolds. Main lubricating oil systems ___________ lubrication for the turbine bearings and the reduction gears. The motor-driven lube oil service pump __________ as standby for the other two pumps.

___10

4. Complete the following questions.

- ___________ the function of the ____________? For receiving fuel oil and filling the fuel oil storage tanks.

- ___________ boiler fuel oil system ____________? They include oil tanks, fuel oil piping, fuel oil pumps and other equipment.

- ___________ the fuel oil service system ________? It consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

- ___________ pumps ___________ oil from one tank to another? The booster and transfer pumps can transfer oil from one tank to another.

- _______ main lubricating oil systems on steam-driving ships ___________ to? They are used to provide lubrication for the turbine bearing and the reduction gears.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. ________________

Explain how the steering engine work? ______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 8

1. Translate the following text into Ukrainian.

The Steering Engine

A telemotor is a motor that is operated by means of remote control. A typical example of the telemotor is the steering engine, which operates the rudder. It is bridge-controlled, which means that it is operated from the bridge by the mate or the helmsman.

The most common type of steering engine consists of a starboard- and port cylinder, in which  rams are actuated by electronic signals that are transmitted when the steering wheel on the bridge is turned. The two rams are connected to each other by a swivel block in a crosshead. The crosshead is fixed to the tiller. This tiller has an opening, called the rudder trunk.

The rudder stock is part of the rudder and goes through rudder trunk. When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of the cylinders to be pushed out and the other ram to be pushed in. This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make an alteration of course.

___30

2. Complete  the sentences below with information from the text

1) When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of ____________

2) This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make_____________

3) It is bridge-controlled, which means that it is operated from the bridge by ____

___10

3. Read the text again and find information to answer these questions.

1. How is called the opening of the tiller?

2. What does the most common type of steering engine consist of ?

3. What happens when the steering-wheel is turned?

4. What is a telemotor?

___10

4. Put these statements in the correct sequence.

- turbines type of very the often on selfoiling the small bearing are

- the carries oil pump and on governor the shaft gear type one the other governor the oil end on

- packing labyrinth type carbon and are two of packing used

- lined of sleeve shell babbit is the with bearing the

- wheel turbine of consist shaft and assemblies rotors

___10

5. Complete the sentences with the correct form of the verb in brackets.

- A turbine __________(consist of) a rotor, a casing and nozzles.

- In some cases reduction gears __________ (require).

- Access openings __________ sometimes (provide) in the casing.

- On impulse turbines, an equalizing hole ______ (drill) through the rotor.

- Turbine rotor ___________ (support, keep) in position by bearings.

- Many turbine-driven auxiliary units ____________ (join) to the turbine by means of a flexible coupling.

- A lot of different designs of flexible couplings ____________ (use).

- These bearings _____________ (have) one or two rings.

- Oil ______________ (pipe) to the bearings on the turbine shaft.

- If the master had given the order earlier, the vessel (not ground)

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _________________

Explain the work of the anchor winch. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 9

1. Translate the following text into Ukrainian.

Main and Auxiliary Turbines

Steam turbines are used for ship propulsion and for driving many of the auxiliary machinery units associated with the propulsion plant, such as lubricating pumps, condensate pumps, feed pumps, circulating pumps, fuel oil pumps, forced draft blowers, and electric generators.

The two principal types of steam turbine ship propulsion plants in use today are the geared-turbine drive and the turbo-electric drive.

In the geared-turbine drive, a reduction gear unit is used as the means for transforming high turbine rpm to a much lower propeller shaft rpm.

In the electric drive, the necessary speed reduction is done electrically. That is the turbine drives a generator at high rpm; and the generator supplies power to drive an electric propulsion motor, which operates in a low rpm range.

In the geared-turbine drive, the propulsion shaft, which extends from the low speed shaft of the reduction gear to the propeller, is supported and held in alignment by the spring bearings and the stern tube bearings.

The axial thrust, acting on the propulsion shaft as a result of the pushing effect of the propeller, is absorbed in the main thrust bearing. In most ships, the main thrust bearing is located at the forward end of the main shaft, within the reduction gear casing. In some very large ships, however, the main shaft thrust bearing is located farther aft in a machinery space or a shaft alley.

Unlike gear-turbine propulsion plants, which have two ahead turbines and an astern element for propulsion shaft, the turbo-electric drive installations have a single turbine unit for each installed shaft. The propulsion unit includes a turbine, main generator, propulsion motor, a direct current generator for supplying excitation current to the generator and propulsion motor, and a propulsion control board.

The basic distinction between turbines concerns the manner in which the steam causes the turbine rotor to move. When the rotor is moved by a push or impulse from a high velocity jet of steam that strikes the blades mounted on the wheel, the turbine is called an impulse turbine. When the rotor is moved by the force of reaction, the turbine is called a reaction turbine.

___20

2. Find the answers to these questions in the text.

- What auxiliary machinery units are directly associated with the propulsion plant operation?

- What is the function of the reduction gear?

- In what units is the rotational speed of a turbine or a propeller shaft expressed?

- When is an electric motor called a propulsion motor?

- What do we call an arrangement that generates electric power?

- What units and arrangements does the turbo-electric propulsion plant include?

- What types of bearings are used in the geared-turbine drive? What are their functions?

- What is the distinction between the two principal types of turbines based upon?

___16

3. Complete the sentences with the correct form of the following verbs to have, to be, to hold, to include, to use, to support, to do.

- A reduction gear __________ to transform high turbine rpm to a much lower propeller shaft rpm.

- The two principal types of steam turbine ship propulsion plants in use today ___________ the geared-turbine drive and the turbo-electric drive.

- In electric drive, the necessary speed reduction __________ electrically.

- The propulsion shaft ____________ and ___________ alignment by the spring bearing and the stern tube bearing.

- In most ships, the main thrust bearing ________________ at the forward end of the main shaft.

- The turbo-electric installations ____ a single turbine unit for each installed shaft.

- The propulsion unit ______________ a turbine, main generator, propulsion motor and a direct current generator.

___14

4. Put the words in the right order to form a question.

1.	Are of driving auxiliary units turbines used ship for and many the machinery propulsion steam for?

2.	A drive does generator at turbine the rpm high?

3.	Power what electric supplies to motor an propulsion unit drive?

4.	Is most main the located ships thrust bearing where in?

5.	The components unit are of the propulsion what?

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ______________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 10

1. Translate the following text into Ukrainian.

There are four main types of marine engine: the steam turbine, the gas turbine, the diesel engine and the marine nuclear plant. Each type has its own practical application.

If a vessel is driven by a steam turbine we call her steam-driven ship.

If a ship is driven by a gas turbine she is called a turbine driven vessel.

If a vessel is driven by a nuclear plant she can be called a nuclear-powered ship or ice-breaker.

The propulsion machinery on a steam-driven ship includes the boilers, the propulsion turbines, the condensers, the reduction gears, and the pumps, blowers, deaerating feed tanks, and other auxiliary machinery components, which serve the major propulsion units.

Turbo-generators and auxiliary condensers are usually located in the propulsion machinery spaces. Other engineering equipment not directly associated with the operation of the propulsion plant may be located in or near the propulsion machinery spaces or in the other parts of the ship.

The propulsion machinery spaces may be arranged in several ways. Some ships have fire rooms (containing boilers and the stations for operating them) and engine rooms (containing main engines and the stations for operating them). Many large ships of recent design have spaces, which are called machinery spaces. Each machinery space contains both the boilers and the main engines.

___20

2. Find the answers to these questions in the text.

- What types of marine engine do you know?

- What are the names of the ships driven by them?

- In what kind of ships is nuclear power applied?

- What are the propulsion machinery units of a steam-driven ship?

- What is the function of the auxiliary machinery components?

- In what sorts of spaces may the propulsion and auxiliary machinery units be located?

- What do we call arrangements used for operating boilers of turbines?

___14

3. Complete the following sentences with the correct form of the verb in brackets. Some verbs are passive.

- These  (be) four main types of marine engine.

- If a vessel (drive) by a steam turbine we  (call) her steam-driven ship.

- The propulsion machinery  (include) the boilers, the propulsion turbines, the condensers, the reduction gears and the pumps, blowers etc.

- Turbo-generators and auxiliary condensers (locate) in the propulsion machinery spaces.

- Some ships (have) fire rooms and engine rooms.

- Each machinery space (contain) both the boilers and the main engines.

- Unless it stops raining, they (not open) the hatches soon.

- If proper safety precautions (take) working aloft can be performed without accidents.

___16

4. Put the words in the right order to form a question.

- Are of main engine the types there marine what?

- The include does units machinery propulsion what?

- Do components what auxiliary serve machinery the units?

- In spaces the arranged machinery severed be propulsion ways may?

- Machinery does each contain what space?

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

- In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

- Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator________________________________________

Explain the work of the steering engine _________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 11

1. Translate the following text into Ukrainian.

The pump

In a reciprocating pump a piston (or plunger) moves up and down and forces the liquid in and out  of the pump chamber by creating a vacuum and a pressure force.

In a single acting reciprocating pump the liquid   is drawn in when the piston goes up, and is forced out when the piston goes down.

In a  double-acting reciprocating pump the upward and downward motions of the  piston cause  simultaneous suction- and  discharge actions.

Double-acting displacement pump

When the piston goes down the lower chamber is emptied through the discharge valve (D2) by   pressure. The other discharge valve (D1) is closed by suction.

The suction valve (S1) is opened by suction, allowing liquid to enter the upper pump chamber. The other suction valve (S2) is closed by pressure. When the piston goes up, the reverse process will take place.

In a gearwheel pump the liquid (fuel oils and lubricating oil) enters through the suction side.

The liquid is then captured between the teeth of the gearwheels (cogs) and is carried around towards the discharge side.

The centrifugal pump only works when it has been immersed in the liquid. In its stationary casing a rotating impeller causes a centrifugal force that will discharge the liquid at high velocity through the discharge line.

___30

2. Complete  the sentences below with information from the text

1) In a single acting reciprocating pump the liquid   is……… when the piston goes up, and is forced out when the piston …

2) The liquid is then captured between the teeth of the gearwheels (cogs) and is ……………

3) In its stationary casing a rotating impeller causes a centrifugal force  …………..

___10

3.Read the text again and find information to answer these questions.

1. What do the upward and downward motions of the piston cause in a  double-acting reciprocating pump ?

2. When does the centrifugal pump only work?

3. How does a piston (or plunger) force the liquid in and out of the pump chamber  in a reciprocating pump?

4.  How does the liquid (fuel oils and lubricating oil) enter in a gearwheel pump?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) Pipes should be led to prevent water lodgement.

2) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

3) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

4) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

5) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

- In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 12

1. Translate the following text into Ukrainian.

The exhaust gas boil

The effective power obtained from heavy fuel oil (HFO) is about 50%   (provided that both engine and engineers perform their tasks well).

This means a heavy loss of energy, which for the major part is wasted in the exhaust gases through the flue and funnel.

These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through an exhaust gas boiler.

The position of the exhaust gas boiler, or waste heat boiler, may be in the engine room or in the funnel.

The idea is quite simple: instead of letting the exhaust gases escape from the engine through the flue and funnel, the gases are first passed through a boiler.

The type of boiler that is used for this purpose is often a so called fire tube boiler.

It has a steam chamber in its hemispherical top.

The boiler shell consists of steel plates that are welded together.

To prevent any thermal loss and to protect the boiler shell from being damaged by the heat, fire bricks are used.

Exhaust gases enter the boiler through the exhaust gas inlet. The exhaust gases heat up banks of fire tubes, which heat up the water in the boiler. The exhaust gases then leave the boiler through the exhaust gas outlet.

The water in the boiler is turned to steam, which may be used for heating purposes. On large vessels the steam may even be used to drive a turbo-generator that will produce electricity that can be used to drive auxiliary engines or for the lighting system.

The exhaust gas boiler is often fitted with an oil-firing installation. In such a case the boiler is called a compound boiler, which means that it can be run both on exhaust gases and by oil firing when the ship is in port and the idle engine does not produce any exhaust gases.

___30

2. Complete  the sentences below with information from the text

1) These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through …

2) The boiler shell consists of …

3) The water in the boiler is turned to …

___10

3. Read the text again and find information to answer these questions.

1. What does a  fire tube boiler have?

2.  What is used to protect the boiler shell from being damaged by the heat?

3.  What do exhaust gases enter the boiler through ?

4.  What do the exhaust gases  leave the boiler through?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

2) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their facesparallel.

3) When the pump is not up to proper speed, the discharge valve should be opened.

4) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

5) Pipes should be led to prevent water lodgement.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain how the steering engine work? _______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 13

1. Translate the following text into Ukrainian.

Electric motors

Aboard vessels the electric motors and hydraulic systems perform most of the hard labour. They are indispensable in daily operations. The maintenance and overhauling of electric motors and appliances is therefore of the utmost importance.

Electric motors and generators are similar in construction.

The main components of an electric motor are the rotor and the stator (or armature).

In an electric motor the rotary motion of the rotor within the stationary stator is employed to actuatepumps, winches, derricks, etc.

Most electric motors are run on alternating current (A/C).

The old-fashioned compound motor combines the advantages of the even more old fashioned  shunt motor and the series motor. The compound motor has a high starting torque and a constant running speed.

Nowadays we know two main types of electric motors: the a-synchronous induction motor, which is widely used aboard ships, and the synchronous induction motor, which is hardly used aboard ships.

The advantage of a-synchronous induction motor is the lack of vulnerable carbon brushes that require frequent maintenance.

Its disadvantages are the large amount of initial current that it requires, and the low starting torque it produces.

In the synchronous induction motor three-phase alternating current, either 220, 380 or 440 volts, passes through coils of copper wire in the stator, creating an alternating magnetic field.

Carbon brushes convey the current to the rotor, which will start to rotate.

The advantages of the synchronous motor are the small amount of initial current it requires, and its high starting torque. However, the synchronous motor is expensive, and the carbon brushes require frequent maintenance.

___30

2. Complete  the sentences below with information from the text

1) The advantage of a-synchronous induction motor is the lack of …

2) Carbon brushes convey the current to the ….

3) The compound motor has a high starting ….

___10

3.Read the text again and find information to answer these questions.

1. What are the main components of an electric motor?

2. What are  two main types of electric motors known nowadays?

3. What are the advantages of the synchronous motor?

4. What is the principle difference between a-synchronous induction motor and the synchronous induction motor?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

2) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

3) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

4) When the pump is not up to proper speed, the discharge valve should be opened.

5) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

- See that the emergency and throttle valves are set open.

- Open all the drains.

- Seal the glands by opening steam valve, if provided. Start condensing plant.

- Open the stop valve slightly, sufficiently to warm the turbine without starting it.

- Open the water connection valves. See also that the cooler is not by-passed if a by-pass is fitted.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _____________

Explain the work of the anchor winch. _________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 14

1. Translate the following text into Ukrainian.

Diesel engines

A Diesel engine is a propulsion plant in which the heat that is generated by internal combustion of a mixture of fuel and air is converted into power.

There are slow speed -, medium speed - and high-speed Diesel engines.

By "speed" is understood the number of revolutions per minute (RPM) the shaft can make.

The slow-speed Diesel engine is applied in large vessels with large engine rooms. These engines are very reliable, but take up a lot of space.

Slow-speed Diesel engines consume Heavy Fuel Oil (HFO), which is an inexpensive fuel. This type of fuel, also called residual oil, is used in medium-speed Diesel engines, too. These engines are favoured in large vessels, e.g. in ferries. Compared to the slow-speed engine the medium-speed engine is more compact. The medium-speed Diesel engine requires more maintenance.

High speed Diesel engines are used in small, highly manoeuvrable vessels. The rated capacity of such an engine is very high, but so is its fuel consumption.

Diesel engines may have cylinders that are placed "in line" or in a bevel (oblique) position. Compared to the in-line engine the advantages of the V-engine are obvious: a V-engine requires only one crankshaft and takes up less space. Therefore the V-engine is very suited to be installed in engine rooms of limited size, contrary to the in-line engine, which cannot be used in small engine rooms due to its height. The in-line engine is not as costly as the V-engine. The construction of the in-line engine is simple, which makes the maintenance and overhauling relatively easy and cheap.

As to the construction of Diesel engines we distinguish two main types: the trunk engine and the crosshead engine. In a trunk engine the piston pin and crank pin are directly connected to each other by the connecting rod. Trunk engines are of compacted build and are mostly medium speed- or high speed engines. Compared to the crosshead engines they do require more maintenance. The crosshead engine is taller because of the crosshead and piston rod, which require more space. This engine is mainly used  in  large vessels.The crosshead serves as a hinging connection between piston rod and connecting rod.

___30

2. Complete  the sentences below with information from the text

1) By “speed” is understood the number of …

2) The construction of the in-line engine is simple, which makes the maintenance…

3) In a trunk engine the piston pin and crank pin are directly …

___10

3. Find the answers to these questions in the text.

1. What type of fuel Slow-speed Diesel engines consume?

2. Where are mostly used High speed Diesel engines are used ?

3. What are the advantages of V-engine?

4. What is the principle difference between trunk and crosshead engines?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) When the pump is not up to proper speed, the discharge valve should be opened.

2 After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

3) Pipes should be led to prevent water lodgement.

4) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

5) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ____________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 15

1. Translate the following text into Ukrainian.

Two-stroke engines and four-stroke engines

The piston in a cylinder makes a reciprocating motion - it goes up and down. The up and down motions of a piston are called strokes. The number of strokes of a piston that form a cycle in a cylinder is two or four. The two-stroke principle is applied in slow speed engines.

Compared with the four-stroke engine, its advantage is that each cylinder is capable of generating more power, because the number of power strokes by the piston is twice as many as with the four-stroke cycle at the same crankshaft speed.

The disadvantage of the two-stroke engine is that, unlike with the four-stroke engine, its effective power is limited due to the fact that its ability to burn fuel is less than in a four-stroke engine. This latter engine is used in smaller vessels with medium-speed or high-speed engines.

The two-stroke cycle begins when the piston is in its Bottom Dead Centre (BDC). Air enters the cylinder through the inlet ports. During the compression stroke the air in the cylinder is compressed while the piston travels upwards. Fuel is admitted to the cylinder at the end of this stroke, either mechanically by fuel pumps, or electrically by a common-rail system. The mixture of air and fuel ignites spontaneously due to the high compression and consequent high temperature in the cylinder.

During the power stroke the piston is pushed down by the "explosion" of the mixture in the cylinder. The reciprocating piston and connecting rod will cause the crankshaft to rotate. Now the camshaft is actuated, so that the rocking lever can open the valve (a pushrod may be inserted as a distance piece between camshaft and rocking lever).

Thus, at the end of the power stroke, the exhaust valves are opened, allowing the exhaust gases to escape into the exhaust gas manifold, after which the remaining exhaust gases are removed out of the cylinder by scavenging air. The scavenging system that is most widely employed nowadays is the uniflow scavenging  system,  because  it  removes  almost all  the  exhaust gases out of the  cylinder.

Cross scavenging- or loop scavenging systems are not effective enough and will leave behind small amounts of gases that will form carbon deposits in the cylinder. Before a new cycle is started, the exhaust valve is closed by the exhaust valve spring or by a hydraulic system.

___30

2.Complete  the sentences below with information from the text

1) Before a new cycle is started, the exhaust valve is closed by…

2) The reciprocating piston and connecting rod will cause …

3) Air enters the cylinder through …

___10

3. Read the text again and find information to answer these questions.

1. What does a stroke mean?

2. When is two-stroke principe applied?

3. What are advantages and disadvantages of two-stroke engines?

4.How are  gases  removed out of the cylinder ?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

- Pipes should be led to prevent water lodgement.

- It should be particularly noted that the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

- It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

- The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

- Horizontal pumps should be laid down upon steel or hard 
wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

4. In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

5. Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator______________________________

Explain the work of the steering engine ______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 16

1. Translate the following text into Ukrainian.

Generators

In order to provide electric power for the lighting system, auxiliaries, cranes, derricks, hatches and for domestic use, the ship is equipped with one or more generators.

Aboard vessels a generator produces electricity of 380 volts or 440 volts. The voltage can be stepped down (reduced) to 220 volts or even 110 volts by a transformer.

A generator produces either alternating current (A/C) or direct current (D/C). A converter is used to convert (change) A/C into D/C. Alternating current changes polarity about 50 times per second ("50 Hertz"). In the U.S.A. generators produce electricity of 60 Hz.

The main parts of a generator are the rotor, the stator and the commutator. The rotor is a rotating electro-magnet. It produces a magnetic field. The rotor rotates within the stator.

Around this stator, or armature, coils of wire are wound. These coils induce ("make") electric current. The process is called induction. The induced current is collected by the commutator. Carbon brushes pass the current to the circuit.

Nowadays the  modern generators do  not  have carbon  brushes  because they  require  regular maintenance. Instead, the rotor is excited by the direct current that is delivered by the stator.

___30

2. Complete  the sentences below with information from the text

1) The voltage can be stepped down (reduced) to 220 volts or even 110 volts by ____

2) A converter is used to ______

3) Instead, the rotor is excited by _______

___10

3. Read the text again and find information to answer these questions.

1. What kind of current does a generator produce?

2. What are the main parts of a generator?

3. Why don’t modern generators have carbon  brushes ?

4. Why is the ship equipped with one or more generators?

___10

4. Put the statements in the correct sequence.

- The system the piping soot main includes only the and turbo-generators steam the blowers.

- For operation many of supply auxiliary and saturated systems units of steam machinery systems steam the.

- Pressure maintained system at the auxiliary in the 15 is psig.

- Includes the condensers piping the system pumps and main auxiliary condensate and.

- System the piping feed the includes pump and feed the boiler main.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

- 5. In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. _________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 17

1. Translate the following text into Ukrainian.

Fuel Oil Systems

Boiler fuel oil Systems on board ship include oil tanks, fuel oil piping, fuel oil pumps, and the equipment used for heating, straining, measuring, and burning the fuel oil.

The fuel oil piping system includes (1) the fuel oil filling and transfer system, and (2) the fuel oil service system. The filling and transfer system is used for, receiving fuel oil and filling the fuel oil storage tanks, filling the fuel oil service tanks, changing the list of the ship by transferring fuel oil between port tanks and starboard tanks, changing the trim of the ship by transferring fuel oil between forward tanks and after tanks.

In general, the filling and transfer system consists of large mains running fore and aft, transfer mains, cross-connections, risers for taking on or discharging fuel oil, fuel oil booster and transfer pumps, lines and manifolds arranged so that the booster and transfer pumps can transfer oil from one tank to another and, when necessary, can deliver oil to the suction side of the fuel service pump.

The fuel oil service system consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

Main Lubricating Oil Systems

Main lubricating oil systems on steam-driven ships provide lubrication for the turbine bearings and the reduction gears. The system usually includes a filling and transfer system, a purifying system, and separate service system for the propulsion unit. On most ships, each lube oil service system includes three displacement pumps:

(1) a shaft-driven pump, attached to and driven by either the propulsion shaft or the quill shaft of the reduction gear, is used as the regular lube oil service pump when the shaft is turning fast enough so that the pump can supply the required oil pressure. (2) The turbine-driven lube oil service pump is used while the ship is getting under way, and is then used as standby at normal speeds. The motor-driven lube oil service pump serves as standby for the other two pumps.

___30

2. Find the answers to these questions in the text.

- What arrangements and equipment do the boiler fuel oil systems include?

- What is the function of the filling and transfer system?

- How can the ship's list or trim be corrected?

- What arrangements allow to transfer oil from one tank to another?

- Which side of a pump is called the suction side?

- What components does the fuel oil service system include?

- What is the English for the system providing lubrication for the turbine bearings?

- What type of pump is used in these systems?

- How are the lube oil pumps driven?

- Which pump is used when the ship is getting under way?

___10

3. Complete the sentences with the correct form of the vebr: to serve, to provide, to consist of, to be used for, to include.

Boiler fuel oil system on board ship _________ oil tanks, fuel oil piping, fuel oil pumps and other equipment. The filling and transfer system ____________ performing many functions. It __________ large mains, transfer mains, cross-connections, risers, fuel oil booster and transfer pumps, lines and manifolds. Main lubricating oil systems ___________ lubrication for the turbine bearings and the reduction gears. The motor-driven lube oil service pump __________ as standby for the other two pumps.

___10

4. Complete the following questions.

- ___________ the function of the ____________? For receiving fuel oil and filling the fuel oil storage tanks.

- ___________ boiler fuel oil system ____________? They include oil tanks, fuel oil piping, fuel oil pumps and other equipment.

- ___________ the fuel oil service system ________? It consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

- ___________ pumps ___________ oil from one tank to another? The booster and transfer pumps can transfer oil from one tank to another.

- _______ main lubricating oil systems on steam-driving ships ___________ to? They are used to provide lubrication for the turbine bearing and the reduction gears.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. ________________

Explain how the steering engine work? ______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 18

1. Translate the following text into Ukrainian.

The Steering Engine

A telemotor is a motor that is operated by means of remote control. A typical example of the telemotor is the steering engine, which operates the rudder. It is bridge-controlled, which means that it is operated from the bridge by the mate or the helmsman.

The most common type of steering engine consists of a starboard- and port cylinder, in which  rams are actuated by electronic signals that are transmitted when the steering wheel on the bridge is turned. The two rams are connected to each other by a swivel block in a crosshead. The crosshead is fixed to the tiller. This tiller has an opening, called the rudder trunk.

The rudder stock is part of the rudder and goes through rudder trunk. When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of the cylinders to be pushed out and the other ram to be pushed in. This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make an alteration of course.

___30

2. Complete  the sentences below with information from the text

1) When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of ____________

2) This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make_____________

3) It is bridge-controlled, which means that it is operated from the bridge by ____

___10

3. Read the text again and find information to answer these questions.

1. How is called the opening of the tiller?

2. What does the most common type of steering engine consist of ?

3. What happens when the steering-wheel is turned?

4. What is a telemotor?

___10

4. Put these statements in the correct sequence.

- turbines type of very the often on selfoiling the small bearing are

- the carries oil pump and on governor the shaft gear type one the other governor the oil end on

- packing labyrinth type carbon and are two of packing used

- lined of sleeve shell babbit is the with bearing the

- wheel turbine of consist shaft and assemblies rotors

___10

5. Complete the sentences with the correct form of the verb in brackets.

- A turbine __________(consist of) a rotor, a casing and nozzles.

- In some cases reduction gears __________ (require).

- Access openings __________ sometimes (provide) in the casing.

- On impulse turbines, an equalizing hole ______ (drill) through the rotor.

- Turbine rotor ___________ (support, keep) in position by bearings.

- Many turbine-driven auxiliary units ____________ (join) to the turbine by means of a flexible coupling.

- A lot of different designs of flexible couplings ____________ (use).

- These bearings _____________ (have) one or two rings.

- Oil ______________ (pipe) to the bearings on the turbine shaft.

- If the master had given the order earlier, the vessel (not ground)

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _________________

Explain the work of the anchor winch. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 19

1. Translate the following text into Ukrainian.

Main and Auxiliary Turbines

Steam turbines are used for ship propulsion and for driving many of the auxiliary machinery units associated with the propulsion plant, such as lubricating pumps, condensate pumps, feed pumps, circulating pumps, fuel oil pumps, forced draft blowers, and electric generators.

The two principal types of steam turbine ship propulsion plants in use today are the geared-turbine drive and the turbo-electric drive.

In the geared-turbine drive, a reduction gear unit is used as the means for transforming high turbine rpm to a much lower propeller shaft rpm.

In the electric drive, the necessary speed reduction is done electrically. That is the turbine drives a generator at high rpm; and the generator supplies power to drive an electric propulsion motor, which operates in a low rpm range.

In the geared-turbine drive, the propulsion shaft, which extends from the low speed shaft of the reduction gear to the propeller, is supported and held in alignment by the spring bearings and the stern tube bearings.

The axial thrust, acting on the propulsion shaft as a result of the pushing effect of the propeller, is absorbed in the main thrust bearing. In most ships, the main thrust bearing is located at the forward end of the main shaft, within the reduction gear casing. In some very large ships, however, the main shaft thrust bearing is located farther aft in a machinery space or a shaft alley.

Unlike gear-turbine propulsion plants, which have two ahead turbines and an astern element for propulsion shaft, the turbo-electric drive installations have a single turbine unit for each installed shaft. The propulsion unit includes a turbine, main generator, propulsion motor, a direct current generator for supplying excitation current to the generator and propulsion motor, and a propulsion control board.

The basic distinction between turbines concerns the manner in which the steam causes the turbine rotor to move. When the rotor is moved by a push or impulse from a high velocity jet of steam that strikes the blades mounted on the wheel, the turbine is called an impulse turbine. When the rotor is moved by the force of reaction, the turbine is called a reaction turbine.

___20

2. Find the answers to these questions in the text.

- What auxiliary machinery units are directly associated with the propulsion plant operation?

- What is the function of the reduction gear?

- In what units is the rotational speed of a turbine or a propeller shaft expressed?

- When is an electric motor called a propulsion motor?

- What do we call an arrangement that generates electric power?

- What units and arrangements does the turbo-electric propulsion plant include?

- What types of bearings are used in the geared-turbine drive? What are their functions?

- What is the distinction between the two principal types of turbines based upon?

___16

3. Complete the sentences with the correct form of the following verbs to have, to be, to hold, to include, to use, to support, to do.

- A reduction gear __________ to transform high turbine rpm to a much lower propeller shaft rpm.

- The two principal types of steam turbine ship propulsion plants in use today ___________ the geared-turbine drive and the turbo-electric drive.

- In electric drive, the necessary speed reduction __________ electrically.

- The propulsion shaft ____________ and ___________ alignment by the spring bearing and the stern tube bearing.

- In most ships, the main thrust bearing ________________ at the forward end of the main shaft.

- The turbo-electric installations ____ a single turbine unit for each installed shaft.

- The propulsion unit ______________ a turbine, main generator, propulsion motor and a direct current generator.

___14

4. Put the words in the right order to form a question.

1.	Are of driving auxiliary units turbines used ship for and many the machinery propulsion steam for?

2.	A drive does generator at turbine the rpm high?

3.	Power what electric supplies to motor an propulsion unit drive?

4.	Is most main the located ships thrust bearing where in?

5.	The components unit are of the propulsion what?

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ______________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 20

1. Translate the following text into Ukrainian.

There are four main types of marine engine: the steam turbine, the gas turbine, the diesel engine and the marine nuclear plant. Each type has its own practical application.

If a vessel is driven by a steam turbine we call her steam-driven ship.

If a ship is driven by a gas turbine she is called a turbine driven vessel.

If a vessel is driven by a nuclear plant she can be called a nuclear-powered ship or ice-breaker.

The propulsion machinery on a steam-driven ship includes the boilers, the propulsion turbines, the condensers, the reduction gears, and the pumps, blowers, deaerating feed tanks, and other auxiliary machinery components, which serve the major propulsion units.

Turbo-generators and auxiliary condensers are usually located in the propulsion machinery spaces. Other engineering equipment not directly associated with the operation of the propulsion plant may be located in or near the propulsion machinery spaces or in the other parts of the ship.

The propulsion machinery spaces may be arranged in several ways. Some ships have fire rooms (containing boilers and the stations for operating them) and engine rooms (containing main engines and the stations for operating them). Many large ships of recent design have spaces, which are called machinery spaces. Each machinery space contains both the boilers and the main engines.

___20

2. Find the answers to these questions in the text.

- What types of marine engine do you know?

- What are the names of the ships driven by them?

- In what kind of ships is nuclear power applied?

- What are the propulsion machinery units of a steam-driven ship?

- What is the function of the auxiliary machinery components?

- In what sorts of spaces may the propulsion and auxiliary machinery units be located?

- What do we call arrangements used for operating boilers of turbines?

___14

3. Complete the following sentences with the correct form of the verb in brackets. Some verbs are passive.

- These  (be) four main types of marine engine.

- If a vessel (drive) by a steam turbine we  (call) her steam-driven ship.

- The propulsion machinery  (include) the boilers, the propulsion turbines, the condensers, the reduction gears and the pumps, blowers etc.

- Turbo-generators and auxiliary condensers (locate) in the propulsion machinery spaces.

- Some ships (have) fire rooms and engine rooms.

- Each machinery space (contain) both the boilers and the main engines.

- Unless it stops raining, they (not open) the hatches soon.

- If proper safety precautions (take) working aloft can be performed without accidents.

___16

4. Put the words in the right order to form a question.

- Are of main engine the types there marine what?

- The include does units machinery propulsion what?

- Do components what auxiliary serve machinery the units?

- In spaces the arranged machinery severed be propulsion ways may?

- Machinery does each contain what space?

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

- In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

- Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator________________________________________

Explain the work of the steering engine _________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 21

1. Translate the following text into Ukrainian.

The pump

In a reciprocating pump a piston (or plunger) moves up and down and forces the liquid in and out  of the pump chamber by creating a vacuum and a pressure force.

In a single acting reciprocating pump the liquid   is drawn in when the piston goes up, and is forced out when the piston goes down.

In a  double-acting reciprocating pump the upward and downward motions of the  piston cause  simultaneous suction- and  discharge actions.

Double-acting displacement pump

When the piston goes down the lower chamber is emptied through the discharge valve (D2) by   pressure. The other discharge valve (D1) is closed by suction.

The suction valve (S1) is opened by suction, allowing liquid to enter the upper pump chamber. The other suction valve (S2) is closed by pressure. When the piston goes up, the reverse process will take place.

In a gearwheel pump the liquid (fuel oils and lubricating oil) enters through the suction side.

The liquid is then captured between the teeth of the gearwheels (cogs) and is carried around towards the discharge side.

The centrifugal pump only works when it has been immersed in the liquid. In its stationary casing a rotating impeller causes a centrifugal force that will discharge the liquid at high velocity through the discharge line.

___30

2. Complete  the sentences below with information from the text

1) In a single acting reciprocating pump the liquid   is……… when the piston goes up, and is forced out when the piston …

2) The liquid is then captured between the teeth of the gearwheels (cogs) and is ……………

3) In its stationary casing a rotating impeller causes a centrifugal force  …………..

___10

3.Read the text again and find information to answer these questions.

1. What do the upward and downward motions of the piston cause in a  double-acting reciprocating pump ?

2. When does the centrifugal pump only work?

3. How does a piston (or plunger) force the liquid in and out of the pump chamber  in a reciprocating pump?

4.  How does the liquid (fuel oils and lubricating oil) enter in a gearwheel pump?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) Pipes should be led to prevent water lodgement.

2) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

3) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

4) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

5) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

- In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 22

1. Translate the following text into Ukrainian.

The exhaust gas boil

The effective power obtained from heavy fuel oil (HFO) is about 50%   (provided that both engine and engineers perform their tasks well).

This means a heavy loss of energy, which for the major part is wasted in the exhaust gases through the flue and funnel.

These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through an exhaust gas boiler.

The position of the exhaust gas boiler, or waste heat boiler, may be in the engine room or in the funnel.

The idea is quite simple: instead of letting the exhaust gases escape from the engine through the flue and funnel, the gases are first passed through a boiler.

The type of boiler that is used for this purpose is often a so called fire tube boiler.

It has a steam chamber in its hemispherical top.

The boiler shell consists of steel plates that are welded together.

To prevent any thermal loss and to protect the boiler shell from being damaged by the heat, fire bricks are used.

Exhaust gases enter the boiler through the exhaust gas inlet. The exhaust gases heat up banks of fire tubes, which heat up the water in the boiler. The exhaust gases then leave the boiler through the exhaust gas outlet.

The water in the boiler is turned to steam, which may be used for heating purposes. On large vessels the steam may even be used to drive a turbo-generator that will produce electricity that can be used to drive auxiliary engines or for the lighting system.

The exhaust gas boiler is often fitted with an oil-firing installation. In such a case the boiler is called a compound boiler, which means that it can be run both on exhaust gases and by oil firing when the ship is in port and the idle engine does not produce any exhaust gases.

___30

2. Complete  the sentences below with information from the text

1) These gases are very hot, and by "capturing" this heat before it escapes into the air, they are passed through …

2) The boiler shell consists of …

3) The water in the boiler is turned to …

___10

3. Read the text again and find information to answer these questions.

1. What does a  fire tube boiler have?

2.  What is used to protect the boiler shell from being damaged by the heat?

3.  What do exhaust gases enter the boiler through ?

4.  What do the exhaust gases  leave the boiler through?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

2) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their facesparallel.

3) When the pump is not up to proper speed, the discharge valve should be opened.

4) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

5) Pipes should be led to prevent water lodgement.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. __________________

Explain how the steering engine work? _______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 23

1. Translate the following text into Ukrainian.

Electric motors

Aboard vessels the electric motors and hydraulic systems perform most of the hard labour. They are indispensable in daily operations. The maintenance and overhauling of electric motors and appliances is therefore of the utmost importance.

Electric motors and generators are similar in construction.

The main components of an electric motor are the rotor and the stator (or armature).

In an electric motor the rotary motion of the rotor within the stationary stator is employed to actuatepumps, winches, derricks, etc.

Most electric motors are run on alternating current (A/C).

The old-fashioned compound motor combines the advantages of the even more old fashioned  shunt motor and the series motor. The compound motor has a high starting torque and a constant running speed.

Nowadays we know two main types of electric motors: the a-synchronous induction motor, which is widely used aboard ships, and the synchronous induction motor, which is hardly used aboard ships.

The advantage of a-synchronous induction motor is the lack of vulnerable carbon brushes that require frequent maintenance.

Its disadvantages are the large amount of initial current that it requires, and the low starting torque it produces.

In the synchronous induction motor three-phase alternating current, either 220, 380 or 440 volts, passes through coils of copper wire in the stator, creating an alternating magnetic field.

Carbon brushes convey the current to the rotor, which will start to rotate.

The advantages of the synchronous motor are the small amount of initial current it requires, and its high starting torque. However, the synchronous motor is expensive, and the carbon brushes require frequent maintenance.

___30

2. Complete  the sentences below with information from the text

1) The advantage of a-synchronous induction motor is the lack of …

2) Carbon brushes convey the current to the ….

3) The compound motor has a high starting ….

___10

3.Read the text again and find information to answer these questions.

1. What are the main components of an electric motor?

2. What are  two main types of electric motors known nowadays?

3. What are the advantages of the synchronous motor?

4. What is the principle difference between a-synchronous induction motor and the synchronous induction motor?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

2) The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

3) Horizontal pumps should be laid down upon steel or hard wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

4) When the pump is not up to proper speed, the discharge valve should be opened.

5) After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

- See that the emergency and throttle valves are set open.

- Open all the drains.

- Seal the glands by opening steam valve, if provided. Start condensing plant.

- Open the stop valve slightly, sufficiently to warm the turbine without starting it.

- Open the water connection valves. See also that the cooler is not by-passed if a by-pass is fitted.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _____________

Explain the work of the anchor winch. _________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 24

1. Translate the following text into Ukrainian.

Diesel engines

A Diesel engine is a propulsion plant in which the heat that is generated by internal combustion of a mixture of fuel and air is converted into power.

There are slow speed -, medium speed - and high-speed Diesel engines.

By "speed" is understood the number of revolutions per minute (RPM) the shaft can make.

The slow-speed Diesel engine is applied in large vessels with large engine rooms. These engines are very reliable, but take up a lot of space.

Slow-speed Diesel engines consume Heavy Fuel Oil (HFO), which is an inexpensive fuel. This type of fuel, also called residual oil, is used in medium-speed Diesel engines, too. These engines are favoured in large vessels, e.g. in ferries. Compared to the slow-speed engine the medium-speed engine is more compact. The medium-speed Diesel engine requires more maintenance.

High speed Diesel engines are used in small, highly manoeuvrable vessels. The rated capacity of such an engine is very high, but so is its fuel consumption.

Diesel engines may have cylinders that are placed "in line" or in a bevel (oblique) position. Compared to the in-line engine the advantages of the V-engine are obvious: a V-engine requires only one crankshaft and takes up less space. Therefore the V-engine is very suited to be installed in engine rooms of limited size, contrary to the in-line engine, which cannot be used in small engine rooms due to its height. The in-line engine is not as costly as the V-engine. The construction of the in-line engine is simple, which makes the maintenance and overhauling relatively easy and cheap.

As to the construction of Diesel engines we distinguish two main types: the trunk engine and the crosshead engine. In a trunk engine the piston pin and crank pin are directly connected to each other by the connecting rod. Trunk engines are of compacted build and are mostly medium speed- or high speed engines. Compared to the crosshead engines they do require more maintenance. The crosshead engine is taller because of the crosshead and piston rod, which require more space. This engine is mainly used  in  large vessels.The crosshead serves as a hinging connection between piston rod and connecting rod.

___30

2. Complete  the sentences below with information from the text

1) By “speed” is understood the number of …

2) The construction of the in-line engine is simple, which makes the maintenance…

3) In a trunk engine the piston pin and crank pin are directly …

___10

3. Find the answers to these questions in the text.

1. What type of fuel Slow-speed Diesel engines consume?

2. Where are mostly used High speed Diesel engines are used ?

3. What are the advantages of V-engine?

4. What is the principle difference between trunk and crosshead engines?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

1) When the pump is not up to proper speed, the discharge valve should be opened.

2 After the pump makes a few strokes and the steam cylinder is clear of water the steam cylinder drains should be closed.

3) Pipes should be led to prevent water lodgement.

4) It    should    be    particularly   noted   that   the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

5) It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ____________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 25

1. Translate the following text into Ukrainian.

Two-stroke engines and four-stroke engines

The piston in a cylinder makes a reciprocating motion - it goes up and down. The up and down motions of a piston are called strokes. The number of strokes of a piston that form a cycle in a cylinder is two or four. The two-stroke principle is applied in slow speed engines.

Compared with the four-stroke engine, its advantage is that each cylinder is capable of generating more power, because the number of power strokes by the piston is twice as many as with the four-stroke cycle at the same crankshaft speed.

The disadvantage of the two-stroke engine is that, unlike with the four-stroke engine, its effective power is limited due to the fact that its ability to burn fuel is less than in a four-stroke engine. This latter engine is used in smaller vessels with medium-speed or high-speed engines.

The two-stroke cycle begins when the piston is in its Bottom Dead Centre (BDC). Air enters the cylinder through the inlet ports. During the compression stroke the air in the cylinder is compressed while the piston travels upwards. Fuel is admitted to the cylinder at the end of this stroke, either mechanically by fuel pumps, or electrically by a common-rail system. The mixture of air and fuel ignites spontaneously due to the high compression and consequent high temperature in the cylinder.

During the power stroke the piston is pushed down by the "explosion" of the mixture in the cylinder. The reciprocating piston and connecting rod will cause the crankshaft to rotate. Now the camshaft is actuated, so that the rocking lever can open the valve (a pushrod may be inserted as a distance piece between camshaft and rocking lever).

Thus, at the end of the power stroke, the exhaust valves are opened, allowing the exhaust gases to escape into the exhaust gas manifold, after which the remaining exhaust gases are removed out of the cylinder by scavenging air. The scavenging system that is most widely employed nowadays is the uniflow scavenging  system,  because  it  removes  almost all  the  exhaust gases out of the  cylinder.

Cross scavenging- or loop scavenging systems are not effective enough and will leave behind small amounts of gases that will form carbon deposits in the cylinder. Before a new cycle is started, the exhaust valve is closed by the exhaust valve spring or by a hydraulic system.

___30

2.Complete  the sentences below with information from the text

1) Before a new cycle is started, the exhaust valve is closed by…

2) The reciprocating piston and connecting rod will cause …

3) Air enters the cylinder through …

___10

3. Read the text again and find information to answer these questions.

1. What does a stroke mean?

2. When is two-stroke principe applied?

3. What are advantages and disadvantages of two-stroke engines?

4.How are  gases  removed out of the cylinder ?

___10

4. Translate   the   following   sentences into Russian, paying attention to the word should:

- Pipes should be led to prevent water lodgement.

- It should be particularly noted that the  necessity   for saving of weight and space for good economy has resulted in high-speed pumps of small dimensions.

- It  is  most  important  that  steps   should  be  taken  to   see that the whole system is thoroughly clean before starting up the pump.

- The system should be tested beforehand to ensure that all suction piping and joints, and the packing glands of valves, are free from air leaks.

- Horizontal pumps should be laid down upon steel or hard 
wood chocks, accurately fitted to ensure that the couplings, with the bolts removed, are in correct alignment and with their faces parallel.

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

4. In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

5. Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator______________________________

Explain the work of the steering engine ______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 26

1. Translate the following text into Ukrainian.

Generators

In order to provide electric power for the lighting system, auxiliaries, cranes, derricks, hatches and for domestic use, the ship is equipped with one or more generators.

Aboard vessels a generator produces electricity of 380 volts or 440 volts. The voltage can be stepped down (reduced) to 220 volts or even 110 volts by a transformer.

A generator produces either alternating current (A/C) or direct current (D/C). A converter is used to convert (change) A/C into D/C. Alternating current changes polarity about 50 times per second ("50 Hertz"). In the U.S.A. generators produce electricity of 60 Hz.

The main parts of a generator are the rotor, the stator and the commutator. The rotor is a rotating electro-magnet. It produces a magnetic field. The rotor rotates within the stator.

Around this stator, or armature, coils of wire are wound. These coils induce ("make") electric current. The process is called induction. The induced current is collected by the commutator. Carbon brushes pass the current to the circuit.

Nowadays the  modern generators do  not  have carbon  brushes  because they  require  regular maintenance. Instead, the rotor is excited by the direct current that is delivered by the stator.

___30

2. Complete  the sentences below with information from the text

1) The voltage can be stepped down (reduced) to 220 volts or even 110 volts by ____

2) A converter is used to ______

3) Instead, the rotor is excited by _______

___10

3. Read the text again and find information to answer these questions.

1. What kind of current does a generator produce?

2. What are the main parts of a generator?

3. Why don’t modern generators have carbon  brushes ?

4. Why is the ship equipped with one or more generators?

___10

4. Put the statements in the correct sequence.

- The system the piping soot main includes only the and turbo-generators steam the blowers.

- For operation many of supply auxiliary and saturated systems units of steam machinery systems steam the.

- Pressure maintained system at the auxiliary in the 15 is psig.

- Includes the condensers piping the system pumps and main auxiliary condensate and.

- System the piping feed the includes pump and feed the boiler main.

___10

5. Translate the following sentences into Ukrainian, paying attention to the Nominative with the    Infinitive construction:

- The thermal efficiencies of gas turbines appear to be quite similar to those of steam power plant of comparable size.

- In larger capacities the gas-turbine plant seems   to have a slight advantage over the diesel engine as far as initial costs are  concerned;   the opposite  appears  to be true   in the  smaller capacities.

- High-pressure condensing turbines generally are considered to be the most efficient of all types of turbines.

- A steam turbine can be classified according to the general direction of the steam flow through the turbine, and the way in which steam expansion is made to take place in the turbine.

- 5. In the   impulse turbine   optimum efficiency is achieved if the speed of the  blading  is made to be approximately  one-half the absolute velocity of the steam at the admission to the blades.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

- If wear of the cylinder liner  and   ring   is to be low,   the surfaces must be well lubricated.

- If broken   rings  are   found in a cylinder, the first thing to do is to examine the ends of the rings for bright spots.

- To determine if the clearance is sufficient, an easy method is to hang the piston assembled complete with its rod and swing the rod by hand to test whether it is free.

- Operation   at   medium   speed   is   continued   long   enough to warm up the engine and permit inspection of all the moving parts, especially the valve gear, to see that everything is working properly.

___10

7 . Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. _________________

Explain the work of the gearwheel pump. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 27

1. Translate the following text into Ukrainian.

Fuel Oil Systems

Boiler fuel oil Systems on board ship include oil tanks, fuel oil piping, fuel oil pumps, and the equipment used for heating, straining, measuring, and burning the fuel oil.

The fuel oil piping system includes (1) the fuel oil filling and transfer system, and (2) the fuel oil service system. The filling and transfer system is used for, receiving fuel oil and filling the fuel oil storage tanks, filling the fuel oil service tanks, changing the list of the ship by transferring fuel oil between port tanks and starboard tanks, changing the trim of the ship by transferring fuel oil between forward tanks and after tanks.

In general, the filling and transfer system consists of large mains running fore and aft, transfer mains, cross-connections, risers for taking on or discharging fuel oil, fuel oil booster and transfer pumps, lines and manifolds arranged so that the booster and transfer pumps can transfer oil from one tank to another and, when necessary, can deliver oil to the suction side of the fuel service pump.

The fuel oil service system consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

Main Lubricating Oil Systems

Main lubricating oil systems on steam-driven ships provide lubrication for the turbine bearings and the reduction gears. The system usually includes a filling and transfer system, a purifying system, and separate service system for the propulsion unit. On most ships, each lube oil service system includes three displacement pumps:

(1) a shaft-driven pump, attached to and driven by either the propulsion shaft or the quill shaft of the reduction gear, is used as the regular lube oil service pump when the shaft is turning fast enough so that the pump can supply the required oil pressure. (2) The turbine-driven lube oil service pump is used while the ship is getting under way, and is then used as standby at normal speeds. The motor-driven lube oil service pump serves as standby for the other two pumps.

___30

2. Find the answers to these questions in the text.

- What arrangements and equipment do the boiler fuel oil systems include?

- What is the function of the filling and transfer system?

- How can the ship's list or trim be corrected?

- What arrangements allow to transfer oil from one tank to another?

- Which side of a pump is called the suction side?

- What components does the fuel oil service system include?

- What is the English for the system providing lubrication for the turbine bearings?

- What type of pump is used in these systems?

- How are the lube oil pumps driven?

- Which pump is used when the ship is getting under way?

___10

3. Complete the sentences with the correct form of the vebr: to serve, to provide, to consist of, to be used for, to include.

Boiler fuel oil system on board ship _________ oil tanks, fuel oil piping, fuel oil pumps and other equipment. The filling and transfer system ____________ performing many functions. It __________ large mains, transfer mains, cross-connections, risers, fuel oil booster and transfer pumps, lines and manifolds. Main lubricating oil systems ___________ lubrication for the turbine bearings and the reduction gears. The motor-driven lube oil service pump __________ as standby for the other two pumps.

___10

4. Complete the following questions.

- ___________ the function of the ____________? For receiving fuel oil and filling the fuel oil storage tanks.

- ___________ boiler fuel oil system ____________? They include oil tanks, fuel oil piping, fuel oil pumps and other equipment.

- ___________ the fuel oil service system ________? It consists of a service main, manifold, fuel oil service pumps, meters, burner manifolds and the burner lines.

- ___________ pumps ___________ oil from one tank to another? The booster and transfer pumps can transfer oil from one tank to another.

- _______ main lubricating oil systems on steam-driving ships ___________ to? They are used to provide lubrication for the turbine bearing and the reduction gears.

___10

5. Translate the following instructions into Ukrainian, paying attention to the verb in the Imperative Mood:

1) Shut the stop valve when the turbine is warmed through.

2) Shut all the drains.

3) Open the exhaust valve.

4) Work the hand oil pump sufficiently to achieve 2—3 Ib./sq. in. pressure at the bearings.

5) Open the stop valve to start the turbine, closing it some what immediately afterwards to prevent the speed rising rapidly. Then bring the turbine up to speed in the course of about two minutes

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The rings must also prevent gas leaking from the cylinder into the crankcase.

2) The working stroke will have to be followed by a compression stroke.

3) If the two bearing valves are to be held secure they must be dovetailed.

4) The only possible solution is for the designer to construct the housing or cylinder strong enough to resist the tightening stresses.

5) To   present a smooth   surface for the piston ring to slide along, several treatments other than honing are in use.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of the double-acting displacement pump. ________________

Explain how the steering engine work? ______________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 28

1. Translate the following text into Ukrainian.

The Steering Engine

A telemotor is a motor that is operated by means of remote control. A typical example of the telemotor is the steering engine, which operates the rudder. It is bridge-controlled, which means that it is operated from the bridge by the mate or the helmsman.

The most common type of steering engine consists of a starboard- and port cylinder, in which  rams are actuated by electronic signals that are transmitted when the steering wheel on the bridge is turned. The two rams are connected to each other by a swivel block in a crosshead. The crosshead is fixed to the tiller. This tiller has an opening, called the rudder trunk.

The rudder stock is part of the rudder and goes through rudder trunk. When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of the cylinders to be pushed out and the other ram to be pushed in. This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make an alteration of course.

___30

2. Complete  the sentences below with information from the text

1) When the steering-wheel is turned, the pressure in the hydraulic system is built up, forcing   the ram in one of ____________

2) This actuates the crosshead with the swivel block and the tiller, causing the rudder to turn and the ship to make_____________

3) It is bridge-controlled, which means that it is operated from the bridge by ____

___10

3. Read the text again and find information to answer these questions.

1. How is called the opening of the tiller?

2. What does the most common type of steering engine consist of ?

3. What happens when the steering-wheel is turned?

4. What is a telemotor?

___10

4. Put these statements in the correct sequence.

- turbines type of very the often on selfoiling the small bearing are

- the carries oil pump and on governor the shaft gear type one the other governor the oil end on

- packing labyrinth type carbon and are two of packing used

- lined of sleeve shell babbit is the with bearing the

- wheel turbine of consist shaft and assemblies rotors

___10

5. Complete the sentences with the correct form of the verb in brackets.

- A turbine __________(consist of) a rotor, a casing and nozzles.

- In some cases reduction gears __________ (require).

- Access openings __________ sometimes (provide) in the casing.

- On impulse turbines, an equalizing hole ______ (drill) through the rotor.

- Turbine rotor ___________ (support, keep) in position by bearings.

- Many turbine-driven auxiliary units ____________ (join) to the turbine by means of a flexible coupling.

- A lot of different designs of flexible couplings ____________ (use).

- These bearings _____________ (have) one or two rings.

- Oil ______________ (pipe) to the bearings on the turbine shaft.

- If the master had given the order earlier, the vessel (not ground)

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) A separate bedplate to carry the crankshaft may be made lighter than if the crankcase is designed to carry the shaft.

2) If the gasket is placed on the cage, it will fall off and, if placed   on the seat at the bottom   of the housing, it is likely to be pinched between the edges of seat and cage.

3) A separate circulating pump is used and the oil or fresh water leaving the pistons passes through coolers and into a large tank to be recirculated,

4) The main   thing to look out when   handling   the   fuel is to keep all dirt and water out of the system.

5) Unlike the steam plant, where the large amount of heat discharged into the condenser is at too low a temperature to be used, the heat in the   exhaust   of a diesel   engine   is at a high temperature, more in line with the  dry stack gases  leaving a boiler than the waste heat of a steam turbine.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work and construction of the exhaust gas boiler _________________

Explain the work of the anchor winch. _______________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 29

1. Translate the following text into Ukrainian.

Main and Auxiliary Turbines

Steam turbines are used for ship propulsion and for driving many of the auxiliary machinery units associated with the propulsion plant, such as lubricating pumps, condensate pumps, feed pumps, circulating pumps, fuel oil pumps, forced draft blowers, and electric generators.

The two principal types of steam turbine ship propulsion plants in use today are the geared-turbine drive and the turbo-electric drive.

In the geared-turbine drive, a reduction gear unit is used as the means for transforming high turbine rpm to a much lower propeller shaft rpm.

In the electric drive, the necessary speed reduction is done electrically. That is the turbine drives a generator at high rpm; and the generator supplies power to drive an electric propulsion motor, which operates in a low rpm range.

In the geared-turbine drive, the propulsion shaft, which extends from the low speed shaft of the reduction gear to the propeller, is supported and held in alignment by the spring bearings and the stern tube bearings.

The axial thrust, acting on the propulsion shaft as a result of the pushing effect of the propeller, is absorbed in the main thrust bearing. In most ships, the main thrust bearing is located at the forward end of the main shaft, within the reduction gear casing. In some very large ships, however, the main shaft thrust bearing is located farther aft in a machinery space or a shaft alley.

Unlike gear-turbine propulsion plants, which have two ahead turbines and an astern element for propulsion shaft, the turbo-electric drive installations have a single turbine unit for each installed shaft. The propulsion unit includes a turbine, main generator, propulsion motor, a direct current generator for supplying excitation current to the generator and propulsion motor, and a propulsion control board.

The basic distinction between turbines concerns the manner in which the steam causes the turbine rotor to move. When the rotor is moved by a push or impulse from a high velocity jet of steam that strikes the blades mounted on the wheel, the turbine is called an impulse turbine. When the rotor is moved by the force of reaction, the turbine is called a reaction turbine.

___20

2. Find the answers to these questions in the text.

- What auxiliary machinery units are directly associated with the propulsion plant operation?

- What is the function of the reduction gear?

- In what units is the rotational speed of a turbine or a propeller shaft expressed?

- When is an electric motor called a propulsion motor?

- What do we call an arrangement that generates electric power?

- What units and arrangements does the turbo-electric propulsion plant include?

- What types of bearings are used in the geared-turbine drive? What are their functions?

- What is the distinction between the two principal types of turbines based upon?

___16

3. Complete the sentences with the correct form of the following verbs to have, to be, to hold, to include, to use, to support, to do.

- A reduction gear __________ to transform high turbine rpm to a much lower propeller shaft rpm.

- The two principal types of steam turbine ship propulsion plants in use today ___________ the geared-turbine drive and the turbo-electric drive.

- In electric drive, the necessary speed reduction __________ electrically.

- The propulsion shaft ____________ and ___________ alignment by the spring bearing and the stern tube bearing.

- In most ships, the main thrust bearing ________________ at the forward end of the main shaft.

- The turbo-electric installations ____ a single turbine unit for each installed shaft.

- The propulsion unit ______________ a turbine, main generator, propulsion motor and a direct current generator.

___14

4. Put the words in the right order to form a question.

1.	Are of driving auxiliary units turbines used ship for and many the machinery propulsion steam for?

2.	A drive does generator at turbine the rpm high?

3.	Power what electric supplies to motor an propulsion unit drive?

4.	Is most main the located ships thrust bearing where in?

5.	The components unit are of the propulsion what?

___10

5. Translate the following sentences into Russian, paying attention to the words that and those:

1) Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

2) In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

3) Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

4) All  marine  diesel  engines,  except those  installed  in  submarines and in ships equipped with diesel electric propulsion, arestarted with compressed air.

5) When heavy, viscous fuels are used approaching those used by steamships, oil heaters will also be required.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

1) The steam turbine consists of a device to convert some of the thermal  energy of the steam into kinetic energy and of a suitable mechanical arrangement to convert  the kinetic energy of the steam into shaftwork.

2) The non-condensing or back-pressure turbine exhausts its steam at a constant pressure to be used for heating or processing purposes.

3) In turbine design the amount of condenser vacuum to be carried must be given due attention, since the vacuum exerts a most important influence upon the turbine economy.

4) The design of a steam turbine plant for a given vessel must depend upon the steam condition chosen, the power to be developed, the reduction gear ratio to he used.

5) Under   normal   condition   at sea   with the diesel engine there is very little to be done except keep eyes and ears open.

___10

7. Give explanations using the general knowledge you have on the subject.

What is a convertor?_______________________________________

What is a transformer? ______________________________________

___20

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

з дисципліни «Англійська мова за професійним спрямуванням»

# спеціальність ЕСЕУ, курс 3, заочна скорочена форма навчання

Variant 30

1. Translate the following text into Ukrainian.

There are four main types of marine engine: the steam turbine, the gas turbine, the diesel engine and the marine nuclear plant. Each type has its own practical application.

If a vessel is driven by a steam turbine we call her steam-driven ship.

If a ship is driven by a gas turbine she is called a turbine driven vessel.

If a vessel is driven by a nuclear plant she can be called a nuclear-powered ship or ice-breaker.

The propulsion machinery on a steam-driven ship includes the boilers, the propulsion turbines, the condensers, the reduction gears, and the pumps, blowers, deaerating feed tanks, and other auxiliary machinery components, which serve the major propulsion units.

Turbo-generators and auxiliary condensers are usually located in the propulsion machinery spaces. Other engineering equipment not directly associated with the operation of the propulsion plant may be located in or near the propulsion machinery spaces or in the other parts of the ship.

The propulsion machinery spaces may be arranged in several ways. Some ships have fire rooms (containing boilers and the stations for operating them) and engine rooms (containing main engines and the stations for operating them). Many large ships of recent design have spaces, which are called machinery spaces. Each machinery space contains both the boilers and the main engines.

___20

2. Find the answers to these questions in the text.

- What types of marine engine do you know?

- What are the names of the ships driven by them?

- In what kind of ships is nuclear power applied?

- What are the propulsion machinery units of a steam-driven ship?

- What is the function of the auxiliary machinery components?

- In what sorts of spaces may the propulsion and auxiliary machinery units be located?

- What do we call arrangements used for operating boilers of turbines?

___14

3. Complete the following sentences with the correct form of the verb in brackets. Some verbs are passive.

- These  (be) four main types of marine engine.

- If a vessel (drive) by a steam turbine we  (call) her steam-driven ship.

- The propulsion machinery  (include) the boilers, the propulsion turbines, the condensers, the reduction gears and the pumps, blowers etc.

- Turbo-generators and auxiliary condensers (locate) in the propulsion machinery spaces.

- Some ships (have) fire rooms and engine rooms.

- Each machinery space (contain) both the boilers and the main engines.

- Unless it stops raining, they (not open) the hatches soon.

- If proper safety precautions (take) working aloft can be performed without accidents.

___16

4. Put the words in the right order to form a question.

- Are of main engine the types there marine what?

- The include does units machinery propulsion what?

- Do components what auxiliary serve machinery the units?

- In spaces the arranged machinery severed be propulsion ways may?

- Machinery does each contain what space?

___10

5. Translate   the   following   sentences into Russian, paying attention to the words that and those:

1.	The steam that is used for heating is not recoverable.

2.	Frames are sometimes called "crankcases" for the reason that the frame walls enclose the cranks and connecting rods.

3. Supercharging is essentially nothing more than forcing air into the engine cylinder at a pressure higher than that of the atmosphere.

- In a two-stroke   engine with crankcase   precombustion the displacement of the scavenging pump is exactly the same as that of the working cylinder.

- Owing to the fact that the maximum pressure occurs at the top of the liner and the minimum at the lower end of the stroke the thickness usually tapers toward the bottom to about one-half that at the top.

___10

6. Translate the following sentences into Ukrainian, paying attention to the function of the Infinitive:

- In   some cases   an impulse turbine is designed to show a pressure difference  across the blading.

- One group of nozzles is sufficient to pass the steam required for all powers up to 5,000 s.h.p.

- Diaphragms are used between successive stages in impulse turbines to act as the nozzle support and to prevent the passage of steam from stage to stage.

- To prevent joint leakage   a groove is cut in both halves of the diaphragm and a composition strip or key inserted and secured.

- After passing through the  high   pressure  or   intermediate pressure casing, the volume of steam becomes too great to be passed through one set of low pressure blades   of reasonable   size,hence the double flow unit is used.

___10

7. Give explanations using the general knowledge you have on the subject.

Explain the work of generator________________________________________

Explain the work of the steering engine _________________________________

___20