МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 1

- Give Ukrainian / Russian equivalents to the following phrases :

To remove the cylinder cover Nos ...

To extract cylinder liners ... pcs

To carry out the inspection of beads for defects ... pcs

To submit for inspection to Register Surveyor

То calibrate the liners before extraction and after reassembling

To check centering

2. Read the text and answer the questions according the text.

Ships require the use of many types of cable and cordage both  for their own security and for the handling of cargo. Three main types are used: chain  cable, steel wires and rope cordage which may be of either natural fibre (e.g. manilla, hemp, cotton or coir) or synthetic fibre (e.g. polypropylene - usually called "polyprop" - nylon or terylene), or a mixture.

Chain cable is made of links of forged iron or steel, the separate lengths of chain being joined together by special links called "shackles". Anchor cables are normally made up in 15 fathom (go-foot) length. Each of these lengths is also known as a "shackle".

Robe is made of fibre or strands, twisted into different lays. Lays commonly found include hawser-laid, shroud-laid, cable-laid and braided.

Different lays are used for wire rope.

  - What is chain cable made of?

  - What do the ships need for their own security and for the handling cargo?

  - What is "a shackle"?

  - What is rope made of?

  - What do lays include?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) What does the master of the m/v “Fizik Lebedev”?

4) In what case should Messrs. J.V.Parsons & Bros. send their representative aboard that vessel?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 2

- Give Ukrainian / Russian equivalents to the following phrases:

Block bearings of cylinders Nos ...

To unbolt the block bearings ... sets

To remove the upper and lower parts of bearings ... sets

To fabricate gaskets ... pcs

To reassemble, to paint-fit and to establish clearances ...

2. Read the text and answer the questions according the text.

The main propulsion comprises two sets of Lindholmenbuilt Pielstick Diesel engines type 18PC 2.5V each with an output of 1 1.700 bhp at 520 rev/min. The NCR at 90% mcr is 11.530 bhp at 500 rev/min. The trial speed at  90 % mcr  is approximately 19.2 knots and the speed with the ship fully laden at 90% mcr, 15% sea margin, being about 19 knots. The fuel consumption is 148 gm/hp plus 5% when burning fuel with a lower calorific value of 10.100 kcal/kg. The main engine is designed to burn fuel oil up to 177 c/s at 50°C, which is equivalent to

1.500 sees fuel oil.

Fuel consumption is as follows:

19 knots fully laden - 67 t/day

19 knots 7.4 m draught - 55 t/day.

All these calculations are on the vessel burning fuel oil of 177 c/s with the fuel heating by means of steam from the exhaust gas economizer or the auxiliary boiler. The main engines are connected to twin screws through horizontally.

1. What does the main propulsion comprise?

2. What is the trial speed?

3. What can you say about fuel consumption?

4. Where are the main engines connected to?

5. The main engine is designed to burn fuel oil up to 200 c/s at 50°C, isn't it?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) What happened to the fireline piping of that ship and what should the repairers do with it?

4) On what conditions should the work under items 1-5 be done?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 3

- Give Ukrainian / Russian equivalents to the following phrases:

Diesel-generator...power... KW

To disconnect piping and fittings

To remove cylinder cover

To take out piston

To extract cylinder liners

To take out main bearings

2. Read the text and answer the questions according the text.

There are two principal forms of corrosion. One is direct chemical attack and mainly occurs in superheaters due to the high metal temperature involved. It can result in pitting or cracking in tube bores, or in scaling or flaking on the gas side of tubes. This form of corrosion also occurs when loss of water circulation causes the metal to overheat in the presence of steam.

The more common form of corrosion found in boilers is the result of electro- chemical attack usually involving acidic water conditions in the presence of dissolved oxygen.

General wastage of the boiler metal due to this form of attack has been virtually eliminated by the use of chemical feed water treatment, but isolated pitting can still occur if the treatment is not operated within the correct limits. It may be found along the water level in the drum, generally as the result of poor shut-down and storage procedures where the boiler is left partly filled with cold water. Pitting along the roof of the drum can result from condensation.

  - How many principal forms of corrosion do you know?

  - Where does the form of corrosion occur?

  - What has been virtually eliminated by the use of chemical feed water treatment?

  - What can occur if the treatment is not operated within the correct limits?

  - Where may it be found?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) Whom are Messrs. Van der Hilfen & Co. asked to send aboard the m/v “Belitsk”?

4) What should be done to the heel block of the cargo derrick?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 4

- Give Ukrainian / Russian equivalents to the following phrases:

To open the turbocharger

After dismantling all parts to be inspected for defects, to take readings

To renew the defective parts

To submit for inspection to Register Surveyor

To reassemble and commission the engine

2. Read the text and answer the questions according the text.

- Which tools can be applied for overhauling older engine and for modern engines?

- Why is it advisable to use hydraulic tightening gear when overhauling modern engines?

- What kind of tools or devices are used\applied to carry out overhauls on older ships?

- In which case do engine foundations and holding down bolts require special attention?

- Is it always possible to keep holding-down bolts tight?

3. Make up questions of all types to one of the sentences from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) What is the diver to do aboard the m/v “Floreshti”?

2) What arrangements does the Captain want his Agents to make with the tug owners?

3) Where did the ship lose her anchor?

4) What words indicate that the Agents should act without delay in this matter?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 5

- Give Ukrainian / Russian equivalents to the following phrases:

To remove covers

To clean the tubes from steam side and sea water side

To renew protectors, gaskets, bolts and nuts

To clean the covers and to apply epoxy composition coating

Condenser to be reassembled and to pressure

To submit for inspection to Register Surveyor

2. Read the text and answer the questions according the text.

- What kind of unpleasant hard jobs are often done by engineers outside the engine room?

- How can unpleasant tasks on board a ship be eliminated?

- What unpleasant jobs have you done onboard?

- What are the essentials of good in-situ maintenance?

- What jobs are most difficult as far as you know from your own experience?

3. Make up questions of all types to one of the sentences from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) When should the m/v “Pula” arrive at Melbourne in accordance with her schedule of sailing?

2) How should the main bearings be repaired?

3) What should be the outside diameter of piston rings, and how many millimetres thick should they be?

4) Of what length should a copper pipe be fitted?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 6

- Translate the following sentences:

The Company_______ herein after referred to as "The Contractor", hereby guarantee BLASCO, Odessa, that:

1. The repairs carried out on the Vessel comply fully with the final documentation as agreed with the Contractor, Owner and ... Register of Shipping.

2. Reliable and trouble free operation of the vessel are guaranteed in normal operation conditions only for those parts and equipment which where the subject of repairs.

2. Read the text and answer the questions according the text.

Ships require the use of many types of cable and cordage both  for their own security and for the handling of cargo. Three main types are used: chain  cable, steel wires and rope cordage which may be of either natural fibre (e.g. manilla, hemp, cotton or coir) or synthetic fibre (e.g. polypropylene - usually called "polyprop" - nylon or terylene), or a mixture.

Chain cable is made of links of forged iron or steel, the separate lengths of chain being joined together by special links called "shackles". Anchor cables are normally made up in 15 fathom (go-foot) length. Each of these lengths is also known as a "shackle".

Robe is made of fibre or strands, twisted into different lays. Lays commonly found include hawser-laid, shroud-laid, cable-laid and braided.

Different lays are used for wire rope.

- What is chain cable made of?

- What do the ships need for their own security and for the handling cargo?

- What is "a shackle"?

- What is rope made of?

- What do lays include?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) What happened to the fireline piping of that ship and what should the repairers do with it?

4) On what conditions should the work under items 1-5 be done?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 7

- Translate the following sentences:

1. All materials and items used for the repair and/or refitting of the vessel comply fully with... Register of Shipping Requirements in respect of their technical characteristics, and their installation and assembly are to procure normal operational conditions of the vessel.

2. The contractor is liable for all defects and deficiencies which may arise due to poor workmanship, defective machinery and equipment, newly installed, incorrect installation or use of defective materials up to the limit of liability specified in the Contract.

2. Read the text and answer the questions according the text.

The main propulsion comprises two sets of Lindholmenbuilt Pielstick Diesel engines type 18PC 2.5V each with an output of 1 1.700 bhp at 520 rev/min. The NCR at 90% mcr is 11.530 bhp at 500 rev/min. The trial speed at  90 % mcr  is approximately 19.2 knots and the speed with the ship fully laden at 90% mcr, 15% sea margin, being about 19 knots. The fuel consumption is 148 gm/hp plus 5% when burning fuel with a lower calorific value of 10.100 kcal/kg. The main engine is designed to burn fuel oil up to 177 c/s at 50°C, which is equivalent to

1.500 sees fuel oil.

Fuel consumption is as follows:

19 knots fully laden - 67 t/day

19 knots 7.4 m draught - 55 t/day.

All these calculations are on the vessel burning fuel oil of 177 c/s with the fuel heating by means of steam from the exhaust gas economizer or the auxiliary boiler. The main engines are connected to twin screws through horizontally.

1. What does the main propulsion comprise?

2. What is the trial speed?

3. What can you say about fuel consumption?

4. Where are the main engines connected to?

5. The main engine is designed to burn fuel oil up to 200 c/s at 50°C, isn't it?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) What does the master of the m/v “Fizik Lebedev”?

4) In what case should Messrs. J.V.Parsons & Bros. send their representative aboard that vessel?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 8

- Translate the following sentences:

1. If defective parts are required to be replaced, such replacement shall to be done or They are to be supplied to the Owner on C.I.F. terms port, in that case installation of the said parts shall be carried out at the Contractor's cost and the guarantee period for replaced devices and equipment is to be counted from the time of their installation.

2. The place where defects discovered during operation of the Vessel and which are subjected to elimination to be repaired is to be mutually agreed between the two parties.

2. Read the text and answer the questions according the text.

There are two principal forms of corrosion. One is direct chemical attack and mainly occurs in superheaters due to the high metal temperature involved. It can result in pitting or cracking in tube bores, or in scaling or flaking on the gas side of tubes. This form of corrosion also occurs when loss of water circulation causes the metal to overheat in the presence of steam.

The more common form of corrosion found in boilers is the result of electro- chemical attack usually involving acidic water conditions in the presence of dissolved oxygen.

General wastage of the boiler metal due to this form of attack has been virtually eliminated by the use of chemical feed water treatment, but isolated pitting can still occur if the treatment is not operated within the correct limits. It may be found along the water level in the drum, generally as the result of poor shut-down and storage procedures where the boiler is left partly filled with cold water. Pitting along the roof of the drum can result from condensation.

- How many principal forms of corrosion do you know?

- Where does the form of corrosion occur?

- What has been virtually eliminated by the use of chemical feed water treatment?

- What can occur if the treatment is not operated within the correct limits?

- Where may it be found?

3. Make up questions of all types to the underlined sentence from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) What is the diver to do aboard the m/v “Floreshti”?

2) What arrangements does the Captain want his Agents to make with the tug owners?

3) Where did the ship lose her anchor?

4) What words indicate that the Agents should act without delay in this matter?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 9

- Give Ukrainian / Russian equivalents to the following phrases:

To remove the cylinder cover Nos ...

To extract cylinder liners ... pcs

To carry out the inspection of beads for defects ... pcs

To submit for inspection to Register Surveyor

То calibrate the liners before extraction and after reassembling

To check centering

2. Read the text and answer the questions according the text.

- Which tools can be applied for overhauling older engine and for modern engines?

- Why is it advisable to use hydraulic tightening gear when overhauling modern engines?

- What kind of tools or devices are used\applied to carry out overhauls on older ships?

- In which case do engine foundations and holding down bolts require special attention?

- Is it always possible to keep holding-down bolts tight?

3. Make up questions of all types to one of the sentences from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) Where and when was letter written?

2) Who was the letter written to?

3) Whom are Messrs. Van der Hilfen & Co. asked to send aboard the m/v “Belitsk”?

4) What should be done to the heel block of the cargo derrick?

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Ділова англійська мова»

# Cпеціальність ЕСЕУ, курс 3 скор., заочна форма навчання

# Variant 10

- Give Ukrainian / Russian equivalents to the following phrases:

Block bearings of cylinders Nos ...

To unbolt the block bearings ... sets

To remove the upper and lower parts of bearings ... sets

To fabricate gaskets ... pcs

To reassemble, to paint-fit and to establish clearances ...

2. Read the text and answer the questions according the text.

- What kind of unpleasant hard jobs are often done by engineers outside the engine room?

- How can unpleasant tasks on board a ship be eliminated?

- What unpleasant jobs have you done onboard?

- What are the essentials of good in-situ maintenance?

- What jobs are most difficult as far as you know from your own experience?

3. Make up questions of all types to one of the sentences from the text.

4. Fulfil grammar task.

5.  Read the letter and answer the questions:

1) When should the m/v “Pula” arrive at Melbourne in accordance with her schedule of sailing?

2) How should the main bearings be repaired?

3) What should be the outside diameter of piston rings, and how many millimetres thick should they be?

4) Of what length should a copper pipe be fitted?