МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 1

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- Racking stresses in a ship can be reduced by fitting a good system of......

  - stringers.

  - deck girders.

  - side girders.

  - bulkheads.

- What stress is the connection of frames and beams at deck level  between the bulkheads designed to resist?

  - Hogging

  - Sagging

  - Racking

  - Pounding

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is low. What will You do?

  - Increase the oil temperature.

  - Start to purify the oil.

  - Decrease the oil temperature.

  - Clean the system and change the oil.

- What could be the reason if the lub oil viscosity increase?

  - Water-leakage to the lub oil system.

  - High lub oil temperature.

  - Heavy fuel oil leakage to the lub oil system.

  - The lub oil viscosity will not increase.

- Ships may only discharge processed bilge water from machinery spaces into the sea if certain very
specific criteria are being met. Which of those listed below is correct?

  - The ship has cin operation as require reg. 16(5) of Anex I

  - The ship has in operation equipment as require reg. 16(5) of Annex I to MARPOL

  - The ship is sailing within a special area

  - The ship is proceeding an route

- Basic procedures for bunkering operations on all ships require that ............................. should personally supervise all bunkering operations

  - the Master

  - the Chief Engineer

  - A responsible person

  - The Second Engineer

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Maintenance onboard.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 2

1. Give English equivalents to the following words and phrases (topic “Bunkering”):

- назначать кого-либо для наблюдения за утечками и переливом;

- снимать показания счетчика;

- отвечать за бункеровку;

- обеспечивать более высокую (низкую) производительность насоса;

- поставщик топлива.

2. Find the correct answer:

- What is distortion of a vessel's structure caused by rolling called?

  - Racking

  - Slamming

  - Pounding

  - Heaving

- Gross tonnage is defined as....

  - earning capacity

  - internal capacity

  - light displacement

  - load displacement

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- What could be the reason if the lub oil viscosity decrease?

  - Diesel oil leakage to the lub oil system.

  - Heavy fuel oil leakage to the lub oil system.

  - Low lub oil temperature.

  - Wrong lub oil pressure.

- A water content greater than 2 % is detected in a lub oil system. What is the best action to do?

  - Continuous purifying the system with the lub oil pumps running.

  - Boil out the water by increasing the temperature.

  - Segregate the oil in a seperate tank for continuous purifying.

  - It is no danger before the water level reach 6-8%.

- Which of the following pumps is not allowed to have a direct overboard connection to the sea?

  - Ballast Water Pumps

  - Engine Room Bilge Pump

  - Cargo Hold Bilge Pump

  - Sludge Pump

- Any discharge of oil into the sea is prohibited, except when some conditions are satisfied. One of these conditions for an oil tanker is that ....

  - the tanker is more than 10 nautical miles from the nearest land.

  - the tanker is more than 25 nautical miles from the nearest land

  - the tanker is more than 50 nautical miles from the nearest land./En route

  - the tanker is more than 100 nautical miles from the nearest land.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 4

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2. Find the correct answer:

- What is the connecting plate between a side frame and a transverse deck beam called?

  - Bracket

  - Gusset

  - Knee

  - Stiffener

- What are bulkhead stiffeners attached to the tank top by?

  - Cleats

  - Brackets

  - Stays

  - Chocks

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- The main reason for the use of cylinder oil is to:

  - To improve combustion efficiency.

  - To protect the piston and liner

  - Lubricate the piston rings.

  - Cool the piston and liner.

- What long term effect will excessively high temperature have on lubricating oil quality?

  - Cause oxydation which increase viscosity.

  - Cause oxydation which reduce viscosity.

  - Evaporates the oil giving high consumption.

  - The oil flashpoint will be changed.

- Which is nearest to the Marpol description of segregated ballast?

  - Ballast water that cannot be pumped through cargo oil piping and for which cargo pumps cannot be used

  - The ballast water introduced in a tank which is completely separated from the cargo or fuel oil system and permanently allocated to ballast

  - The clean ballast water which can only be pumped via segregated lines by the clean ballast pump

  - The ballast carried in the clean ballast tank, pumped via the segregated ballast pump

- Which description given is nearest to the description of clean ballast as defined by Marpol?

  - Its oil content is NIL and consequently would not leave any traces upon the surface of the water or emulsion beneath the water surface.

  - If discharged from a stationary ship in clean calm water on a clear day will not produce visible traces of oil on the water surface nor emulsion below

  - Its oil content is no more than 50 ppm and will not produce visible traces on adjoining shorelines, nor emulsion in the sea.

  - Its oil content is no more than 100 ppm and it will not produce visible traces on the shore lines.

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 4

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the uppermost continuous hull plating called?

  - Sheer Strake

  - Garboard Strake

  - Stealer Plate

  - Stringer Plate

- Moulded beam is the measurement taken to the.....

  - inside of the plating.

  - outside of the plating

  - inside of the frames.

  - inside of the ceiling

- When the temperature of lubricating oil  is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

- During operation of the main engine, the heat absorbed by lubricating oil is removed by the:

  - Sump vents.

  - Scavenge air cooler.

  - Any of the above.

  - Lube oil cooler

- Which would be the most suitable place to obtain an oil sample for analysis?

  - From the oil cooler drain.

  - From the lub. oil pump suction line.

  - From the system pipeline.

  - From the filter drain.

- If diesel fuel contamination occurs in the crankcase oil of an auxiliary engine, the oil viscosity will:

  - Stay the same

  - Nothing happens.

  - Decrease.

  - Increase.

- What is the meaning of "slop tank" as defined by Marpol?

  - A tank used for tank cleaning provided with heating coils

  - Any tank where slops or sludges are collected

  - The two most aftward fitted wing tanks or the most aft ward fitted centre tank

  - A tank specifically designed for the collection of tank drainings, washings or other oily mixtures

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- In accordance with SOLAS and some international regulations, a very important function shall be strictly tested prior to arrival and departure. In addition regular drills shall be performed ensuring proper operating function and cooperation of the functions. What function?

a) Auxiliary machinery and functions

b) None of the mentioned alternatives

c) Steering gear and functions

d) Propulsion machinery and functions

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Marine pollution.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 5

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2. Find the correct answer:

- Where are panting stresses for a ship most severe?

  - Forward

  - Amidships

  - Aft

  - Between the bulkheads

- What is the weight a vessel can carry called?

  - Load Displacement

  - Gross Tonnage

  - Registered Tonnage

  - Deadweight

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is ok.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- If water contamination occurs in the crankcase oil of an auxiliary engine the oil viscosity will:

  - Increase.

  - Decrease.

  - Stay at the same.

  - Nothing happens.

- What type of lubricating oils are generally used in auxiliary engines?

  - Alkaline.

  - Synthetic

  - It is not important.

  - Detergent.

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- Who can certify true copies of Oil Record Books?

  - Port Authorities

  - A Notary Public

  - The Officers of the Flag State

  - The Master

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the dialogue between the Bunker Supplier and the Chief Engineer on the topic (about 8-10 lines).

Bunkering.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 6

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- When a vessel is hogging the keel will be in........

  - sheer.

  - tension.

  - torsion.

  - compression.

- What is the vertical distance top to top between the load line marks S and F?

  - Summer Allowance

  - Tropical Allowance

  - Dock Water Allowance

  - Fresh Water Allowance

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- In a Burmeister & Wain main engine, at what point is cylinder oil injected?

  - As the piston passes "The Lubrication Points" in an upward direction.

  - Just before "Top Dead Centre".

  - As the piston passes "The Lubrication Points" in a downward direction.

  - Just before "Bottom Dead Centre".

- What could cause engine oil to get a "Milky" colour?

  - Water contamination.

  - Fuel contamination

  - Overheating.

  - The oil is too cold.

- Where is the Oil Record Book to be kept on board?

  - In the Master's cabin

  - On the bridge

  - In the ship's office

  - At a place where it is readily available

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 7

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the type of framing shown in the ship cross section?

  - Transverse

  - Longitudinal

  - Combination

  - Web

- A General Arrangement drawing gives details of a vessel's....

  - construction sequence

  - tank capacities

  - hydrostatic information

  - layout

- What is the minimum number of transverse bulkheads required for a vessel with machinery aft?

  - Three

  - Four

  - Five

  - Six

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is OK.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- What can happen with lub oil cooled piston if you stop the lub. oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealings in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is high. What will You do?

  - Increase the cooling water temperature for the oil.

  - Increase the lub oil pressure.

  - Clean the system and change the oil.

  - Decrease the cooling water temperature for the oil.

- Which of the following is not a "Special Area" according to MARPOL Regulation 10?

  - Mediterranean Sea area including the Black Sea

  - Baltic Sea area

  - Red Sea area

  - Caribbean Sea area

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Translate the following sentences and explain the grammar rules which you used (topic “Repairs”):

- Ремонт судна должен выполняться в соответствии с технической документацией.

- Стоимость дополнительных работ должна быть взаимно согласована.

- Если подрядчик не сможет выполнить ремонт в срок, он обязан заплатить владельцу штраф.

- Замените новой наружную обшивку.

- Заменяемые части должны быть поставлены верфью.

- Вся работа, выполненная в сухом доке, должна быть предъявлена Регистру для осмотра.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 8

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2.  Find the correct answer:

- The lower end of the stem bar of a ship is attached to a........

  - Stringer

  - Frame

  - Beam

  - keel plate

- What stresses are the vessel's structure forward of the collision bulkhead stiffened to resist?

  - Panting & Pounding

  - Panting & Racking

  - Water Pressure

  - Hogging & Sagging

- Additional bottom stiffening is required immediately aft side of the collision bulkhead to resist........

  - racking.

  - panting.

  - pounding.

  - hogging.

- Why is it so important to keep the lub oil viscosity within safe limits?

  - To keep the oil protective film between moving metal  surfaces .

  - The lub oil viscosity has no influence on the oil's lubrication ability.

  - To keep the additives in good condition.

  - It is not important

- Total Base Number ( TBN ) is important as a measure of neutralizing ability against strong  Acids. What will You do if this TBN base number is reduced to a low level?

  - Renew lub oil in the system.

  - Purify the oil system.

  - Send one lub oil sample to new analysis.

  - Nothing.

- What can happen with lub oil cooled piston if you stop the lub oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealing in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- The entries in the Oil Record Book must be............

  - in the language of the Captain and in English or French.

  - in the language of the crew and in English or French

  - in the official language of the Flag State and in English or French.

  - in any language comprehensible by the Port State Control officers.

- Bunkering procedures for all ships require that ..................... communication should be maintained with the shore/barge staff in charge of the transfer.

  - Continuous

  - Frequent

  - Occasional

  - intermittent

- In accordance with SOLAS and some international regulations, certain important functions shall always be tested prior to arrival and departure. What functions?

  - All mentioned function

  - Steering machinery and control functions

  - Generator plants and control functions

  - Propulsion machinery and control functions

- When the temperature of lubricating oil is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

4. Translate the following sentences (topic “In-situ repairs”):

- Важно, чтобы все основные параметры непрерывно проверялись.

- Переборка поршней главных двигателей и рамовых подшипников проводится на борту механиками, когда судно находится в порту достаточно долго.

- Цилиндр и каждая пара подшипников вскрываются и проверяются инспектором классификационного общества, если он есть в данном порту.

- Переборка насосов и вспомогательных двигателей проводится в порту аналогично.

- Если установлены запасные насосы, один из каждой пары можно перебирать в море.

- В море может также производиться периодическая переборка запасных клапанов.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 9

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2.  Find the correct answer:

- At sea, water is self-drained from an exposed deck by.........

  - discharge pipes.

  - side scuttles.

  - freeing ports.

  - drain valves.

- What is a drawing used to identify individual strakes and plates called?

  - Construction Drawing

  - Sheer Profile

  - Shell Expansion Plan

  - Body Plan

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- What differential pressure would you expect to find across an engine lub.oil filter?

  - 0.2 Bar

  - Zero

  - 1.2 Bar

  - 4.0 Bar

- You notice the sump level in an engine has increased and no new oil has been added. What action would you take?

  - Stop the engine and look for a fuel or water leak.

  - Leave it the same.

  - Drain some oil from the engine.

  - Reduced the load, and look after fuel or water leak.

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release how ever caused.

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 10

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2.  Find the correct answer:

- In ideal conditions, the forward distance a propeller will move in one revolution of the shaft is the:

  - Diameter

  - Pitch

  - Slip

  - circumference

- What is the hinge on which a rudder turns called?

  - Bolt

  - Pin

  - Gudgeon

  - Coupling

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- Oil analysis results show a high level of tin. What could be the cause of this?

  - Fuel contamination.

  - Too high lub oil temperature.

  - Bearing damage.

  - Water contamination.

- Which PPM is allowed for discharging of "Bilge Water" overboard?

  - 15 PPM

  - 100 PPM

  - 50 PPM

  - 0 PPM

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release however caused.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

4. Write the information on the topic (about 200 words).   Be ready to speak on it.

Marine pollution.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 11

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- Racking stresses in a ship can be reduced by fitting a good system of......

  - stringers.

  - deck girders.

  - side girders.

  - bulkheads.

- What stress is the connection of frames and beams at deck level  between the bulkheads designed to resist?

  - Hogging

  - Sagging

  - Racking

  - Pounding

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is low. What will You do?

  - Increase the oil temperature.

  - Start to purify the oil.

  - Decrease the oil temperature.

  - Clean the system and change the oil.

- What could be the reason if the lub oil viscosity increase?

  - Water-leakage to the lub oil system.

  - High lub oil temperature.

  - Heavy fuel oil leakage to the lub oil system.

  - The lub oil viscosity will not increase.

- Ships may only discharge processed bilge water from machinery spaces into the sea if certain very
specific criteria are being met. Which of those listed below is correct?

  - The ship has cin operation as require reg. 16(5) of Anex I

  - The ship has in operation equipment as require reg. 16(5) of Annex I to MARPOL

  - The ship is sailing within a special area

  - The ship is proceeding an route

- Basic procedures for bunkering operations on all ships require that ............................. should personally supervise all bunkering operations

  - the Master

  - the Chief Engineer

  - A responsible person

  - The Second Engineer

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Maintenance onboard.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 12

1. Give English equivalents to the following words and phrases (topic “Bunkering”):

- назначать кого-либо для наблюдения за утечками и переливом;

- снимать показания счетчика;

- отвечать за бункеровку;

- обеспечивать более высокую (низкую) производительность насоса;

- поставщик топлива.

2. Find the correct answer:

- What is distortion of a vessel's structure caused by rolling called?

  - Racking

  - Slamming

  - Pounding

  - Heaving

- Gross tonnage is defined as....

  - earning capacity

  - internal capacity

  - light displacement

  - load displacement

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- What could be the reason if the lub oil viscosity decrease?

  - Diesel oil leakage to the lub oil system.

  - Heavy fuel oil leakage to the lub oil system.

  - Low lub oil temperature.

  - Wrong lub oil pressure.

- A water content greater than 2 % is detected in a lub oil system. What is the best action to do?

  - Continuous purifying the system with the lub oil pumps running.

  - Boil out the water by increasing the temperature.

  - Segregate the oil in a seperate tank for continuous purifying.

  - It is no danger before the water level reach 6-8%.

- Which of the following pumps is not allowed to have a direct overboard connection to the sea?

  - Ballast Water Pumps

  - Engine Room Bilge Pump

  - Cargo Hold Bilge Pump

  - Sludge Pump

- Any discharge of oil into the sea is prohibited, except when some conditions are satisfied. One of these conditions for an oil tanker is that ....

  - the tanker is more than 10 nautical miles from the nearest land.

  - the tanker is more than 25 nautical miles from the nearest land

  - the tanker is more than 50 nautical miles from the nearest land./En route

  - the tanker is more than 100 nautical miles from the nearest land.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 14

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2. Find the correct answer:

- What is the connecting plate between a side frame and a transverse deck beam called?

  - Bracket

  - Gusset

  - Knee

  - Stiffener

- What are bulkhead stiffeners attached to the tank top by?

  - Cleats

  - Brackets

  - Stays

  - Chocks

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- The main reason for the use of cylinder oil is to:

  - To improve combustion efficiency.

  - To protect the piston and liner

  - Lubricate the piston rings.

  - Cool the piston and liner.

- What long term effect will excessively high temperature have on lubricating oil quality?

  - Cause oxydation which increase viscosity.

  - Cause oxydation which reduce viscosity.

  - Evaporates the oil giving high consumption.

  - The oil flashpoint will be changed.

- Which is nearest to the Marpol description of segregated ballast?

  - Ballast water that cannot be pumped through cargo oil piping and for which cargo pumps cannot be used

  - The ballast water introduced in a tank which is completely separated from the cargo or fuel oil system and permanently allocated to ballast

  - The clean ballast water which can only be pumped via segregated lines by the clean ballast pump

  - The ballast carried in the clean ballast tank, pumped via the segregated ballast pump

- Which description given is nearest to the description of clean ballast as defined by Marpol?

  - Its oil content is NIL and consequently would not leave any traces upon the surface of the water or emulsion beneath the water surface.

  - If discharged from a stationary ship in clean calm water on a clear day will not produce visible traces of oil on the water surface nor emulsion below

  - Its oil content is no more than 50 ppm and will not produce visible traces on adjoining shorelines, nor emulsion in the sea.

  - Its oil content is no more than 100 ppm and it will not produce visible traces on the shore lines.

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 14

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the uppermost continuous hull plating called?

  - Sheer Strake

  - Garboard Strake

  - Stealer Plate

  - Stringer Plate

- Moulded beam is the measurement taken to the.....

  - inside of the plating.

  - outside of the plating

  - inside of the frames.

  - inside of the ceiling

- When the temperature of lubricating oil  is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

- During operation of the main engine, the heat absorbed by lubricating oil is removed by the:

  - Sump vents.

  - Scavenge air cooler.

  - Any of the above.

  - Lube oil cooler

- Which would be the most suitable place to obtain an oil sample for analysis?

  - From the oil cooler drain.

  - From the lub. oil pump suction line.

  - From the system pipeline.

  - From the filter drain.

- If diesel fuel contamination occurs in the crankcase oil of an auxiliary engine, the oil viscosity will:

  - Stay the same

  - Nothing happens.

  - Decrease.

  - Increase.

- What is the meaning of "slop tank" as defined by Marpol?

  - A tank used for tank cleaning provided with heating coils

  - Any tank where slops or sludges are collected

  - The two most aftward fitted wing tanks or the most aft ward fitted centre tank

  - A tank specifically designed for the collection of tank drainings, washings or other oily mixtures

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- In accordance with SOLAS and some international regulations, a very important function shall be strictly tested prior to arrival and departure. In addition regular drills shall be performed ensuring proper operating function and cooperation of the functions. What function?

a) Auxiliary machinery and functions

b) None of the mentioned alternatives

c) Steering gear and functions

d) Propulsion machinery and functions

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Marine pollution.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 15

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2. Find the correct answer:

- Where are panting stresses for a ship most severe?

  - Forward

  - Amidships

  - Aft

  - Between the bulkheads

- What is the weight a vessel can carry called?

  - Load Displacement

  - Gross Tonnage

  - Registered Tonnage

  - Deadweight

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is ok.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- If water contamination occurs in the crankcase oil of an auxiliary engine the oil viscosity will:

  - Increase.

  - Decrease.

  - Stay at the same.

  - Nothing happens.

- What type of lubricating oils are generally used in auxiliary engines?

  - Alkaline.

  - Synthetic

  - It is not important.

  - Detergent.

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- Who can certify true copies of Oil Record Books?

  - Port Authorities

  - A Notary Public

  - The Officers of the Flag State

  - The Master

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the dialogue between the Bunker Supplier and the Chief Engineer on the topic (about 8-10 lines).

Bunkering.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 16

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- When a vessel is hogging the keel will be in........

  - sheer.

  - tension.

  - torsion.

  - compression.

- What is the vertical distance top to top between the load line marks S and F?

  - Summer Allowance

  - Tropical Allowance

  - Dock Water Allowance

  - Fresh Water Allowance

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- In a Burmeister & Wain main engine, at what point is cylinder oil injected?

  - As the piston passes "The Lubrication Points" in an upward direction.

  - Just before "Top Dead Centre".

  - As the piston passes "The Lubrication Points" in a downward direction.

  - Just before "Bottom Dead Centre".

- What could cause engine oil to get a "Milky" colour?

  - Water contamination.

  - Fuel contamination

  - Overheating.

  - The oil is too cold.

- Where is the Oil Record Book to be kept on board?

  - In the Master's cabin

  - On the bridge

  - In the ship's office

  - At a place where it is readily available

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 17

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the type of framing shown in the ship cross section?

  - Transverse

  - Longitudinal

  - Combination

  - Web

- A General Arrangement drawing gives details of a vessel's....

  - construction sequence

  - tank capacities

  - hydrostatic information

  - layout

- What is the minimum number of transverse bulkheads required for a vessel with machinery aft?

  - Three

  - Four

  - Five

  - Six

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is OK.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- What can happen with lub oil cooled piston if you stop the lub. oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealings in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is high. What will You do?

  - Increase the cooling water temperature for the oil.

  - Increase the lub oil pressure.

  - Clean the system and change the oil.

  - Decrease the cooling water temperature for the oil.

- Which of the following is not a "Special Area" according to MARPOL Regulation 10?

  - Mediterranean Sea area including the Black Sea

  - Baltic Sea area

  - Red Sea area

  - Caribbean Sea area

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Translate the following sentences and explain the grammar rules which you used (topic “Repairs”):

- Ремонт судна должен выполняться в соответствии с технической документацией.

- Стоимость дополнительных работ должна быть взаимно согласована.

- Если подрядчик не сможет выполнить ремонт в срок, он обязан заплатить владельцу штраф.

- Замените новой наружную обшивку.

- Заменяемые части должны быть поставлены верфью.

- Вся работа, выполненная в сухом доке, должна быть предъявлена Регистру для осмотра.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 18

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2.  Find the correct answer:

- The lower end of the stem bar of a ship is attached to a........

  - Stringer

  - Frame

  - Beam

  - keel plate

- What stresses are the vessel's structure forward of the collision bulkhead stiffened to resist?

  - Panting & Pounding

  - Panting & Racking

  - Water Pressure

  - Hogging & Sagging

- Additional bottom stiffening is required immediately aft side of the collision bulkhead to resist........

  - racking.

  - panting.

  - pounding.

  - hogging.

- Why is it so important to keep the lub oil viscosity within safe limits?

  - To keep the oil protective film between moving metal  surfaces .

  - The lub oil viscosity has no influence on the oil's lubrication ability.

  - To keep the additives in good condition.

  - It is not important

- Total Base Number ( TBN ) is important as a measure of neutralizing ability against strong  Acids. What will You do if this TBN base number is reduced to a low level?

  - Renew lub oil in the system.

  - Purify the oil system.

  - Send one lub oil sample to new analysis.

  - Nothing.

- What can happen with lub oil cooled piston if you stop the lub oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealing in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- The entries in the Oil Record Book must be............

  - in the language of the Captain and in English or French.

  - in the language of the crew and in English or French

  - in the official language of the Flag State and in English or French.

  - in any language comprehensible by the Port State Control officers.

- Bunkering procedures for all ships require that ..................... communication should be maintained with the shore/barge staff in charge of the transfer.

  - Continuous

  - Frequent

  - Occasional

  - intermittent

- In accordance with SOLAS and some international regulations, certain important functions shall always be tested prior to arrival and departure. What functions?

  - All mentioned function

  - Steering machinery and control functions

  - Generator plants and control functions

  - Propulsion machinery and control functions

- When the temperature of lubricating oil is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

4. Translate the following sentences (topic “In-situ repairs”):

- Важно, чтобы все основные параметры непрерывно проверялись.

- Переборка поршней главных двигателей и рамовых подшипников проводится на борту механиками, когда судно находится в порту достаточно долго.

- Цилиндр и каждая пара подшипников вскрываются и проверяются инспектором классификационного общества, если он есть в данном порту.

- Переборка насосов и вспомогательных двигателей проводится в порту аналогично.

- Если установлены запасные насосы, один из каждой пары можно перебирать в море.

- В море может также производиться периодическая переборка запасных клапанов.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 19

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2.  Find the correct answer:

- At sea, water is self-drained from an exposed deck by.........

  - discharge pipes.

  - side scuttles.

  - freeing ports.

  - drain valves.

- What is a drawing used to identify individual strakes and plates called?

  - Construction Drawing

  - Sheer Profile

  - Shell Expansion Plan

  - Body Plan

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- What differential pressure would you expect to find across an engine lub.oil filter?

  - 0.2 Bar

  - Zero

  - 1.2 Bar

  - 4.0 Bar

- You notice the sump level in an engine has increased and no new oil has been added. What action would you take?

  - Stop the engine and look for a fuel or water leak.

  - Leave it the same.

  - Drain some oil from the engine.

  - Reduced the load, and look after fuel or water leak.

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release how ever caused.

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 20

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2.  Find the correct answer:

- In ideal conditions, the forward distance a propeller will move in one revolution of the shaft is the:

  - Diameter

  - Pitch

  - Slip

  - circumference

- What is the hinge on which a rudder turns called?

  - Bolt

  - Pin

  - Gudgeon

  - Coupling

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- Oil analysis results show a high level of tin. What could be the cause of this?

  - Fuel contamination.

  - Too high lub oil temperature.

  - Bearing damage.

  - Water contamination.

- Which PPM is allowed for discharging of "Bilge Water" overboard?

  - 15 PPM

  - 100 PPM

  - 50 PPM

  - 0 PPM

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release however caused.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

4. Write the information on the topic (about 200 words).   Be ready to speak on it.

Marine pollution.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 21

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- Racking stresses in a ship can be reduced by fitting a good system of......

  - stringers.

  - deck girders.

  - side girders.

  - bulkheads.

- What stress is the connection of frames and beams at deck level  between the bulkheads designed to resist?

  - Hogging

  - Sagging

  - Racking

  - Pounding

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is low. What will You do?

  - Increase the oil temperature.

  - Start to purify the oil.

  - Decrease the oil temperature.

  - Clean the system and change the oil.

- What could be the reason if the lub oil viscosity increase?

  - Water-leakage to the lub oil system.

  - High lub oil temperature.

  - Heavy fuel oil leakage to the lub oil system.

  - The lub oil viscosity will not increase.

- Ships may only discharge processed bilge water from machinery spaces into the sea if certain very
specific criteria are being met. Which of those listed below is correct?

  - The ship has cin operation as require reg. 16(5) of Anex I

  - The ship has in operation equipment as require reg. 16(5) of Annex I to MARPOL

  - The ship is sailing within a special area

  - The ship is proceeding an route

- Basic procedures for bunkering operations on all ships require that ............................. should personally supervise all bunkering operations

  - the Master

  - the Chief Engineer

  - A responsible person

  - The Second Engineer

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Maintenance onboard.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 22

1. Give English equivalents to the following words and phrases (topic “Bunkering”):

- назначать кого-либо для наблюдения за утечками и переливом;

- снимать показания счетчика;

- отвечать за бункеровку;

- обеспечивать более высокую (низкую) производительность насоса;

- поставщик топлива.

2. Find the correct answer:

- What is distortion of a vessel's structure caused by rolling called?

  - Racking

  - Slamming

  - Pounding

  - Heaving

- Gross tonnage is defined as....

  - earning capacity

  - internal capacity

  - light displacement

  - load displacement

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- What could be the reason if the lub oil viscosity decrease?

  - Diesel oil leakage to the lub oil system.

  - Heavy fuel oil leakage to the lub oil system.

  - Low lub oil temperature.

  - Wrong lub oil pressure.

- A water content greater than 2 % is detected in a lub oil system. What is the best action to do?

  - Continuous purifying the system with the lub oil pumps running.

  - Boil out the water by increasing the temperature.

  - Segregate the oil in a seperate tank for continuous purifying.

  - It is no danger before the water level reach 6-8%.

- Which of the following pumps is not allowed to have a direct overboard connection to the sea?

  - Ballast Water Pumps

  - Engine Room Bilge Pump

  - Cargo Hold Bilge Pump

  - Sludge Pump

- Any discharge of oil into the sea is prohibited, except when some conditions are satisfied. One of these conditions for an oil tanker is that ....

  - the tanker is more than 10 nautical miles from the nearest land.

  - the tanker is more than 25 nautical miles from the nearest land

  - the tanker is more than 50 nautical miles from the nearest land./En route

  - the tanker is more than 100 nautical miles from the nearest land.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 24

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2. Find the correct answer:

- What is the connecting plate between a side frame and a transverse deck beam called?

  - Bracket

  - Gusset

  - Knee

  - Stiffener

- What are bulkhead stiffeners attached to the tank top by?

  - Cleats

  - Brackets

  - Stays

  - Chocks

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- The main reason for the use of cylinder oil is to:

  - To improve combustion efficiency.

  - To protect the piston and liner

  - Lubricate the piston rings.

  - Cool the piston and liner.

- What long term effect will excessively high temperature have on lubricating oil quality?

  - Cause oxydation which increase viscosity.

  - Cause oxydation which reduce viscosity.

  - Evaporates the oil giving high consumption.

  - The oil flashpoint will be changed.

- Which is nearest to the Marpol description of segregated ballast?

  - Ballast water that cannot be pumped through cargo oil piping and for which cargo pumps cannot be used

  - The ballast water introduced in a tank which is completely separated from the cargo or fuel oil system and permanently allocated to ballast

  - The clean ballast water which can only be pumped via segregated lines by the clean ballast pump

  - The ballast carried in the clean ballast tank, pumped via the segregated ballast pump

- Which description given is nearest to the description of clean ballast as defined by Marpol?

  - Its oil content is NIL and consequently would not leave any traces upon the surface of the water or emulsion beneath the water surface.

  - If discharged from a stationary ship in clean calm water on a clear day will not produce visible traces of oil on the water surface nor emulsion below

  - Its oil content is no more than 50 ppm and will not produce visible traces on adjoining shorelines, nor emulsion in the sea.

  - Its oil content is no more than 100 ppm and it will not produce visible traces on the shore lines.

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 24

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the uppermost continuous hull plating called?

  - Sheer Strake

  - Garboard Strake

  - Stealer Plate

  - Stringer Plate

- Moulded beam is the measurement taken to the.....

  - inside of the plating.

  - outside of the plating

  - inside of the frames.

  - inside of the ceiling

- When the temperature of lubricating oil  is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

- During operation of the main engine, the heat absorbed by lubricating oil is removed by the:

  - Sump vents.

  - Scavenge air cooler.

  - Any of the above.

  - Lube oil cooler

- Which would be the most suitable place to obtain an oil sample for analysis?

  - From the oil cooler drain.

  - From the lub. oil pump suction line.

  - From the system pipeline.

  - From the filter drain.

- If diesel fuel contamination occurs in the crankcase oil of an auxiliary engine, the oil viscosity will:

  - Stay the same

  - Nothing happens.

  - Decrease.

  - Increase.

- What is the meaning of "slop tank" as defined by Marpol?

  - A tank used for tank cleaning provided with heating coils

  - Any tank where slops or sludges are collected

  - The two most aftward fitted wing tanks or the most aft ward fitted centre tank

  - A tank specifically designed for the collection of tank drainings, washings or other oily mixtures

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- In accordance with SOLAS and some international regulations, a very important function shall be strictly tested prior to arrival and departure. In addition regular drills shall be performed ensuring proper operating function and cooperation of the functions. What function?

a) Auxiliary machinery and functions

b) None of the mentioned alternatives

c) Steering gear and functions

d) Propulsion machinery and functions

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Marine pollution.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 25

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2. Find the correct answer:

- Where are panting stresses for a ship most severe?

  - Forward

  - Amidships

  - Aft

  - Between the bulkheads

- What is the weight a vessel can carry called?

  - Load Displacement

  - Gross Tonnage

  - Registered Tonnage

  - Deadweight

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is ok.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- If water contamination occurs in the crankcase oil of an auxiliary engine the oil viscosity will:

  - Increase.

  - Decrease.

  - Stay at the same.

  - Nothing happens.

- What type of lubricating oils are generally used in auxiliary engines?

  - Alkaline.

  - Synthetic

  - It is not important.

  - Detergent.

- According to Marpol, what is the meaning of the term "oil fuel"?

  - A cargo of heavy fuel oil , intermediate fuel or diesel oil

  - A cargo of refined oil for combustible purpose in thermal machinery or boilers

  - A cargo of refined oil with flashpoint above 60 degree C

  - Any oil used on board in connection with the propulsion and auxiliary machinery.

- Who can certify true copies of Oil Record Books?

  - Port Authorities

  - A Notary Public

  - The Officers of the Flag State

  - The Master

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate  immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

- What are the compartments to be regularly checked with regard to water and oil leakages?

  - All those listed in the other alternatives

  - Bilge wells

  - Tank top in the engine room.

  - Bilge tank(s)

4. Write the dialogue between the Bunker Supplier and the Chief Engineer on the topic (about 8-10 lines).

Bunkering.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 26

- Give English equivalents to the following words and phrases (topic “Pollution”):

- международная конвенция по предотвращению загрязнения с судов;

- контролировать сброс нефти;

- обеспечивать необходимую остойчивость;

- устройство для приема нефтесодержащих вод;

- обязательный ежегодный осмотр.

2. Find the correct answer:

- When a vessel is hogging the keel will be in........

  - sheer.

  - tension.

  - torsion.

  - compression.

- What is the vertical distance top to top between the load line marks S and F?

  - Summer Allowance

  - Tropical Allowance

  - Dock Water Allowance

  - Fresh Water Allowance

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- TBN - total base number from a lube oil test, indicates the:

  - Amount of contaminants in the lubricating oil.

  - Amount of organic acids in the oil.

  - Alkalinity of the lubricating oil.

  - Flash point.

- In a Burmeister & Wain main engine, at what point is cylinder oil injected?

  - As the piston passes "The Lubrication Points" in an upward direction.

  - Just before "Top Dead Centre".

  - As the piston passes "The Lubrication Points" in a downward direction.

  - Just before "Bottom Dead Centre".

- What could cause engine oil to get a "Milky" colour?

  - Water contamination.

  - Fuel contamination

  - Overheating.

  - The oil is too cold.

- Where is the Oil Record Book to be kept on board?

  - In the Master's cabin

  - On the bridge

  - In the ship's office

  - At a place where it is readily available

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- One of the Main Engine cooling pumps is down for maintenance. The rest of the cooling pumps are operating on full capacity with minimum margins on the maximum temperature limit. What will be the correct precaution or action to take at the end of the day?

  - Operate the engine room on manual mode until the cooling pump is in operation, switched in stand-by mode.

  - Trust that the alarm function and automatic temperature regulation will assure safe operation for UMS-mode during the night, switching to UMS-mode as normal

  - Arrange frequent checks of the condition during the night, operating on UMS-mode

  - Arrange with a rating "on watch" in the engine room.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

In-situ repairs.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 27

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2. Find the correct answer:

- What is the type of framing shown in the ship cross section?

  - Transverse

  - Longitudinal

  - Combination

  - Web

- A General Arrangement drawing gives details of a vessel's....

  - construction sequence

  - tank capacities

  - hydrostatic information

  - layout

- What is the minimum number of transverse bulkheads required for a vessel with machinery aft?

  - Three

  - Four

  - Five

  - Six

- An excessive pressure drop across a lube oil strainer could indicate:

  - That the strainer needs cleaning.

  - That the relief valve is stuck open.

  - That the strainer is OK.

  - Any of the above.

- Water is best removed from lubricating oil by:

  - Silica gel cartridges.

  - Pressure filter.

  - Paper filter.

  - Centrifugal separator.

- What action indicates that lube oil has been diluted by fuel oil?

  - A change in the viscosity of the mixture.

  - Lube oil discharging from the waste water outlet of the lube oil purifier.

  - Water discharging from the waste water outlet of the lube oil purifier.

  - A change in the lube oil pressure.

- What can happen with lub oil cooled piston if you stop the lub. oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealings in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- You have checked the oil viscosity in an auxiliary engine with the viscosity test kit. The result shows that the viscosity is high. What will You do?

  - Increase the cooling water temperature for the oil.

  - Increase the lub oil pressure.

  - Clean the system and change the oil.

  - Decrease the cooling water temperature for the oil.

- Which of the following is not a "Special Area" according to MARPOL Regulation 10?

  - Mediterranean Sea area including the Black Sea

  - Baltic Sea area

  - Red Sea area

  - Caribbean Sea area

- What rules and regulations are regulating the watch keeping routines in the engine room?

  - Both STCW and Class rules

  - International Standard for Training and Watch keeping (STCW)

  - Class Rules

  - SOLAS

4. Translate the following sentences and explain the grammar rules which you used (topic “Repairs”):

- Ремонт судна должен выполняться в соответствии с технической документацией.

- Стоимость дополнительных работ должна быть взаимно согласована.

- Если подрядчик не сможет выполнить ремонт в срок, он обязан заплатить владельцу штраф.

- Замените новой наружную обшивку.

- Заменяемые части должны быть поставлены верфью.

- Вся работа, выполненная в сухом доке, должна быть предъявлена Регистру для осмотра.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 28

- Give English equivalents to the following words and phrases (topic “Maintenance”):

- Вредное воздействие;

- Температурные перепады;

- Износоустойчивая поверхность;

- Предотвращать коррозию;

- Удалять остатки старого покрытия.

2.  Find the correct answer:

- The lower end of the stem bar of a ship is attached to a........

  - Stringer

  - Frame

  - Beam

  - keel plate

- What stresses are the vessel's structure forward of the collision bulkhead stiffened to resist?

  - Panting & Pounding

  - Panting & Racking

  - Water Pressure

  - Hogging & Sagging

- Additional bottom stiffening is required immediately aft side of the collision bulkhead to resist........

  - racking.

  - panting.

  - pounding.

  - hogging.

- Why is it so important to keep the lub oil viscosity within safe limits?

  - To keep the oil protective film between moving metal  surfaces .

  - The lub oil viscosity has no influence on the oil's lubrication ability.

  - To keep the additives in good condition.

  - It is not important

- Total Base Number ( TBN ) is important as a measure of neutralizing ability against strong  Acids. What will You do if this TBN base number is reduced to a low level?

  - Renew lub oil in the system.

  - Purify the oil system.

  - Send one lub oil sample to new analysis.

  - Nothing.

- What can happen with lub oil cooled piston if you stop the lub oil pump immediately after You stop the engine?

  - Nothing happens.

  - The oil flashpoint will be changed.

  - You save the oil sealing in the piston.

  - Overheating of the piston, and oil coke deposits in the cooling chambers.

- The entries in the Oil Record Book must be............

  - in the language of the Captain and in English or French.

  - in the language of the crew and in English or French

  - in the official language of the Flag State and in English or French.

  - in any language comprehensible by the Port State Control officers.

- Bunkering procedures for all ships require that ..................... communication should be maintained with the shore/barge staff in charge of the transfer.

  - Continuous

  - Frequent

  - Occasional

  - intermittent

- In accordance with SOLAS and some international regulations, certain important functions shall always be tested prior to arrival and departure. What functions?

  - All mentioned function

  - Steering machinery and control functions

  - Generator plants and control functions

  - Propulsion machinery and control functions

- When the temperature of lubricating oil is lowered, an increase will occur in the:

  - Viscosity.

  - Flash point.

  - Concentration of contaminants.

  - Pour point.

4. Translate the following sentences (topic “In-situ repairs”):

- Важно, чтобы все основные параметры непрерывно проверялись.

- Переборка поршней главных двигателей и рамовых подшипников проводится на борту механиками, когда судно находится в порту достаточно долго.

- Цилиндр и каждая пара подшипников вскрываются и проверяются инспектором классификационного общества, если он есть в данном порту.

- Переборка насосов и вспомогательных двигателей проводится в порту аналогично.

- Если установлены запасные насосы, один из каждой пары можно перебирать в море.

- В море может также производиться периодическая переборка запасных клапанов.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 29

- Give English equivalents to the following words and phrases (topic “Repairs. Contract”):

- Заключать контракт;

- Выполнить заказ;

- Обеспечить запасными частями;

- Начало ремонта;

- Соответствовать техническим условиям контракта.

2.  Find the correct answer:

- At sea, water is self-drained from an exposed deck by.........

  - discharge pipes.

  - side scuttles.

  - freeing ports.

  - drain valves.

- What is a drawing used to identify individual strakes and plates called?

  - Construction Drawing

  - Sheer Profile

  - Shell Expansion Plan

  - Body Plan

- TAN, total acid number is the:

  - Total content of organic and inorganic acids in the oil.

  - Amount of remaining alkalinity in the oil.

  - Total content of inorganic acids in the oil.

  - Total content of organic acids in the oil.

- A decrease in the viscosity of the lubricating oil in a trunk engine may be due to dilution by:

  - Diesel oil.

  - Cylinder oil.

  - Water.

  - Heavy fuel oil.

- What differential pressure would you expect to find across an engine lub.oil filter?

  - 0.2 Bar

  - Zero

  - 1.2 Bar

  - 4.0 Bar

- You notice the sump level in an engine has increased and no new oil has been added. What action would you take?

  - Stop the engine and look for a fuel or water leak.

  - Leave it the same.

  - Drain some oil from the engine.

  - Reduced the load, and look after fuel or water leak.

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release how ever caused.

- For any engine room operated in UMS-mode, defined checks are to be carried out prior to switching to UMS-mode. Is it the Engineer on Duty's responsibility to have the full control of condition of any check items prior to switching to UMS-mode and leaving the engine room, or may he leave the engine room for a short period (coffee break, lunch etc.) trusting the alarms and automatic functions are sufficient for safe operation?

  - A described UMS-mode check shall always be carried out prior to leaving the engine room, irrespective of the time he will be absent.

  - Ok for 15-20 minutes without a proper check

  - Ok for one hour without a proper check

  - Ok for a short period as long as one rating is left "on watch"

- Prior to the lunch break, the Engineer on Duty observes that the operating generator set has an output of 90%. With regard to operation of the generator, what is the - most important - assumption(s) for him to check before switching to UMS-mode?

  - Visually checking the auxiliary engine that there are no leaks or other obvious operating failures

  - All parameters as listed under the other alternatives.

  - That a secondary auxiliary set is switched to automatic standby mode

  - That a secondary auxiliary set is switched to automatic standby mode

4. Write the information on the topic (about 200 words).

Be ready to speak on it.

Corrosion.

МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

Державний університет інфраструктури та технологій

ДУНАЙСЬКИЙ ФАКУЛЬТЕТ МОРСЬКОГО ТА РІЧКОВОГО ТРАНСПОРТУ

# МОДУЛЬНА КОНТРОЛЬНА РОБОТА

З дисципліни «Англійська мова за професійним спрямуванням»

# Cпеціальність ЕСЕУ, курс 4, заочна форма навчання

# Variant 40

1. Give English equivalents to the following words and phrases (topic “Ordering ship supplies”):

- у нас кончились запасы;

- мы собираемся заказать притирочное масло;

- какова предварительная стоимость?

- низкосортное топливо;

- присадки антикоррозийные.

2.  Find the correct answer:

- In ideal conditions, the forward distance a propeller will move in one revolution of the shaft is the:

  - Diameter

  - Pitch

  - Slip

  - circumference

- What is the hinge on which a rudder turns called?

  - Bolt

  - Pin

  - Gudgeon

  - Coupling

- If a leak occurs in a lube oil cooler, the water cannot mix with the oil because the:

  - Oil pressure is greater than the water pressure.

  - By-pass valve will open.

  - Cooling pump will automatically stop.

  - Outlet valve will automatically close

- The output pressure of a diesel engine lubricating oil pump is regulated by:

  - A relief valve.

  - A metering valve.

  - An orifice.

  - A variable speed pump drive.

- Oil analysis results show a high level of tin. What could be the cause of this?

  - Fuel contamination.

  - Too high lub oil temperature.

  - Bearing damage.

  - Water contamination.

- Which PPM is allowed for discharging of "Bilge Water" overboard?

  - 15 PPM

  - 100 PPM

  - 50 PPM

  - 0 PPM

- Garbage disposal regulations outside special areas specify that dunnage, lining and packing materials that float......

  - may not be thrown overboard at any time.

  - may be thrown overboard outside 12 miles offshore.

  - may be thrown overboard outside 25 miles offshore.

  - may be thrown overboard outside 4 miles offshore.

- Pumping, emptying or emitting from a ship.

  - What does "discharge" means in relation to harmful substances as defined by Marpol?

  - Spilling, leaking or escaping from the ship.

  - Dumping or disposal from the ship.

  - Any release however caused.

- During a routine sea passage the cooling water high temperature alarm system for one of the auxiliary engines is out of function and blocked. What will be the appropriate immediate action?

  - All the actions listed in the other alternatives

  - Have the failure immediately repaired

  - Switch to another generator set (that is available)

  - If the failure is in conflict with pre-requisitional electric power supply, ensure manual operation mode until the failure is corrected

- During execution of a standard UMS-check, the Engineer on Duty is experiencing a "low water level" alarm in a cooling water expansion tank. What will be the appropriate immediate action?

  - Correct water level is obtained by refilling the cooling system, and the entire system is checked for leakages

  - The set point for the water level in the expansion tank is temporarily readjusted for later action.

  - The alarm function is blocked for later action

  - The alarm function is blocked and the engine room is changed to manual mode.

4. Write the information on the topic (about 200 words).   Be ready to speak on it.

Marine pollution.